Bt848 Decoder Class API

- INTRODUCTION

This document describes the functions developed to manipulate the decoder registers of the Bt848. Other registers, such as scaling, color, PCI, etc., are not supported in the decoder class API. All the Bt848 decoder register functions are listed in the following sections.

- Bt848 DECODER Registers Functions

Device Status Register (DSTATUS)

BYTE Bt848Decoder::GetDeviceStatusReg ( void )

Purpose:	Obtain device status register value

Input:	None

Output:	None

Return:	Value of status register

bool Bt848Decoder::IsVideoPresent ( void )

Purpose:	Detect if video is present

Input:	None

Output:	None

Return:	true if video present; else false

bool Bt848Decoder::IsDeviceInHLock ( void )

Purpose:	Detect if device is in H-lock

Input:	None

Output:	None

Return:	true if device in H-lock; else false

bool Bt848Decoder::IsEvenField ( void )

Purpose:	Reflect whether an even or odd field is being decoded

Input:	None

Output:	None

Return:	true if even field; else false

bool Bt848Decoder::Is525LinesVideo ( void )

Purpose:	Check to see if we are dealing with 525 lines video signal

Input:	None

Output:	None

Return:	true if 525 lines detected; else false (assume 625 lines)

bool Bt848Decoder::IsCrystal0Selected ( void )

Purpose:	Reflect whether XTAL0 or XTAL1 is selected

Input:	None

Output:	None

Return:	true if XTAL0 selected; else false (XTAL1 selected)

bool Bt848Decoder::IsLumaOverflow ( void )

Purpose:	Indicates if luma ADC overflow

Input:	None

Output:	None

Return:	true if luma ADC overflow; else false

void Bt848Decoder::ResetLumaOverflow( void )

Purpose:	Reset luma ADC overflow bit

Input:	None

Output:	None

Return:	None

bool Bt848Decoder::IsChromaOverflow ( void )

Purpose:	Indicates if chroma ADC overflow

Input:	None

Output:	None

Return:	true if chroma ADC overflow; else false

void Bt848Decoder::ResetChromaOverflow( void )

Purpose:	Reset chroma ADC overflow bit

Input:	None

Output:	None

Return:	None

Input Format Register (IFORM)

ErrorCode Bt848Decoder::SetVideoInput( Connector source )

Purpose:	Select which connector as input

Input:	Connector source - ConSVideo, ConTuner, ConComposite

Output:	None

Return:	Fail if error in parameter, else Success

int Bt848Decoder::GetVideoInput ( void )

Purpose:	Get which connector is input

Input:	None

Output:	None

Return:	Video source - ConSVideo, ConTuner, ConComposite

ErrorCode Bt848Decoder::SetCrystal (Crystal crystalNo )

Purpose:	Select which crystal as input

Input:	Crystal crystalNo

XT0	- Crystal_XT0

XT1	- Crystal_XT1

Auto select	- Crystal_AutoSelect

Output:	None

Return:	Fail if error in parameter, else Success

int Bt848Decoder::GetCrystal ( void )

Purpose:	Get which crystal is input

Input:	None

Output:	None

Return:	Crystal number

XT0	- Crystal_XT0

XT1	- Crystal_XT1

Auto select	- Crystal_AutoSelect

ErrorCode Bt848Decoder::SetVideoFormat( VideoFormat format )

Purpose:	Set video format

Input:	VideoFormat format

Auto format	- VFormat_AutoDetect

NTSC (M)	- VFormat_NTSC

PAL (B, D, G, H, I)	- VFormat_PAL_BDGHI

PAL (M)	- VFormat_PAL_M

PAL(N)	- VFormat_PAL_N

SECAM	- VFormat_SECAM

Output:	None

Return:	Fail if error in parameter, else Success

Note:	This function also sets the AGCDelay (ADELAY) and BrustDelay (BDELAY) 		registers

int Bt848Decoder::GetVideoFormat ( void )

Purpose:	Obtain video format

Input:	None

Output:	None

Return:	Video format

Auto format	- VFormat_AutoDetect

NTSC (M)	- VFormat_NTSC

PAL (B, D, G, H, I)	- VFormat_PAL_BDGHI

PAL (M)	- VFormat_PAL_M

PAL(N)	- VFormat_PAL_N

SECAM	- VFormat_SECAM

Temporal Decimation Register (TDEC)

ErrorCode Bt848Decoder::SetRate( bool fields, VidField even, int rate )

Purpose:	Set frames or fields rate

Input:	bool fields	- true for fields, false for frames

VidField even	- true to start decimation with even field, false odd

int  rate	- decimation rate: frames (1-50/60); fields(1-25/30)

Output:	None

Return:	Fail if error in parameter, else Success

Brightness Control Register (BRIGHT)

ErrorCode Bt848Decoder::SetBrightness ( int param )

Purpose:	Set video brightness

Input:	int param - parameter value (0-255; default 128)

Output:	None

Return:	Fail if error in parameter, else Success

int Bt848Decoder::GetBrightness ( void )

Purpose:	Obtain brightness value

Input:	None

Output:	None

Return:	Brightness parameter (0-255)

Miscellaneous Control Register (E_CONTROL, O_CONTROL)

void Bt848Decoder::SetLumaNotchFilter ( bool mode )

Purpose:	Enable/Disable luma notch filter

Input:	bool mode - true = Enable; false = Disable

Output:	None

Return:	None

bool Bt848Decoder::IsLumaNotchFilter ( void )

Purpose:	Check if luma notch filter is enable or disable

Input:	None

Output:	None

Return:	true = Enable; false = Disable

void Bt848Decoder::SetCompositeVideo ( bool mode )

Purpose:	Select composite or Y/C component video

Input:	bool mode - true = Composite; false = Y/C Component

Output:	None

Return:	None

bool Bt848Decoder::IsCompositeVideo ( void )

Purpose:	Check if composite or Y/C component video is selected

Input:	None

Output:	None

Return:	true = Composite; false = Y/C Component

void Bt848Decoder::SetLumaDecimation ( bool mode )

Purpose:	Enable/Disable luma decimation filter

Input:	bool mode - true = Enable; false = Disable

Output:	None

Return:	None

bool Bt848Decoder::IsLumaDecimation ( void )

Purpose:	Check if luma decimation filter is enable or disable

Input:	None

Output:	None

Return:	true = Enable; false = Disable

void Bt848Decoder::SetCbFirst ( bool mode )

Purpose:	Control whether the first pixel of a line is a Cb or Cr pixel

Input:	bool mode - true = Normal Cb, Cr order; false = Invert Cb, Cr order

Output:	None

Return:	None

bool Bt848Decoder::IsCbFirst ( void )

Purpose:	Check if the first pixel of a line is a Cb or Cr pixel

Input:	None

Output:	None

Return:	true = Normal Cb, Cr order, false = Invert Cb, Cr order

Luma Gain Register (CON_MSB, CONTRAST_LO)

ErrorCode Bt848Decoder::SetContrast ( int param )

Purpose:	Set video contrast

Input:	int param - parameter value (0-255; default 128)

Output:	None

Return:	Fail if error in parameter, else Success

int Bt848Decoder::GetContrast ( void )

Purpose:	Obtain contrast value

Input:	None

Output:	None

Return:	Contrast parameter (0-255)

Chroma Gain Register (SAT_U_MSB, SAT_V_MSB, SAT_U_LO, SAT_V_LO)

ErrorCode Bt848Decoder::SetSaturation ( int param )

Purpose:	Set color saturation by modifying U and V values

Input:	int param - parameter value (0-255; default 128)

Output:	None

Return:	Fail if error in parameter, else Success

int Bt848Decoder::GetSaturation ( void )

Purpose:	Obtain saturation value

Input:	None

Output:	None

Return:	Saturation parameter (0-255)

Hue Control Register (HUE)

ErrorCode Bt848Decoder::SetHue ( int param)

Purpose:	Set video hue value

Input:	int param - parameter value (0-255; default 128)

Output:	None

Return:	Fail if error in parameter, else Success

int Bt848Decoder::GetHue ( void )

Purpose:	Obtain hue value

Input:	None

Output:	None

Return:	Hue parameter (0-255)

SC Loop Control Register (E_SCLOOP, O_SCLOOP)

void Bt848Decoder::SetChromaAGC ( bool mode )

Purpose:Enable/Disable Chroma AGC compensation

Input:	bool mode - true = Enable, false = Disable

Output:	None

Return:	None

bool Bt848Decoder::IsChromaAGC ( void )

Purpose:	Check if Chroma AGC compensation is enable or disable

Input:	None

Output:	None

Return:	true = Enable, false = Disable

void Bt848Decoder::SetLowColorAutoRemoval ( bool mode )

Purpose:	Enable/Disable low color detection and removal

Input:	bool mode - true = Enable, false = Disable

Output:	None

Return:	None

bool Bt848Decoder::IsLowColorAutoRemoval ( void )

Purpose:	Check if low color detection and removal is enable or disable

Input:	None

Output:	None

Return:	true = Enable, false = Disable

ErrorCode Bt848Decoder::SetHorizontalFilter( HorizFilter hFilter )

Purpose:	Control the configuration of the optional 6-tap Horizontal Low-Pass filter

Input:	Which filter to use

Auto Format	- HFilter_AutoFormat

CIF	- HFilter_CIF

QCIF	- HFilter_QCIF

ICON	- HFilter_ICON

Output:	None

Return:	Fail if error in parameter, else Success

int Bt848Decoder::GetHorizontalFilter ( void )

Purpose:	Get the configuration of the optional 6-tap Horizontal Low-Pass filter

Input:	None

Output:	None

Return:	Which filter is using

Auto Format	- HFilter_AutoFormat

CIF	- HFilter_CIF

QCIF	- HFilter_QCIF

ICON	- HFilter_ICON

Output Format Register (OFORM)

void Bt848Decoder::SetFullOutputRange ( bool mode )

Purpose:	Enable/Disable full output range

Input:	bool mode - true = Enable, false = Disable

Output:	None

Return:	None

bool Bt848Decoder::IsFullOutputRange ( void )

Purpose:	Check if full output range is enable or disable

Input:	None

Output:	None

Return:	true = Enable, false = Disable

ErrorCode Bt848Decoder::SetLumaCoring( CoringLevel cLevel )

Purpose:	Set luminance level such that luminance signal is truncated to zero if below 			this level

Input:	Luma coring level

No coring	- Coring_None

8	- Coring_8

16	- Coring_16

32	- Coring_32

Output:	None

Return:	Fail if error in parameter, else Success

int Bt848Decoder::GetLumaCoring ( void )

Purpose:	Get luminance level such that luminance signal is truncated to zero if below 			this level

Input:	None

Output:	None

Return:	Luma coring level

No coring	- Coring_None

8	- Coring_8

16	- Coring_16

32	- Coring_32

Vertical Scaling Register (E_VSCALE_HI, O_VSCALE_HI)

void Bt848Decoder::SetChromaComb( bool mode )

Purpose:	Enable/Disable chroma comb

Input:	bool mode - true = Enable, false = Disable

Output:	None

Return:	None

bool Bt848Decoder::IsChromaComb( void )

Purpose:	Check if chroma comb is enable or disable

Input:	None

Output:	None

Return:	true = Enable, false = Disable

AGC Delay Register (ADELAY)

void Bt848Decoder::SetAGCDelay ( BYTE value )

Purpose:	Set AGC Delay register

Input:	BYTE value - Value to be set to

Output:	None

Return:	None

int Bt848Decoder::GetAGCDelay ( void )

Purpose:	Get AGC Delay register

Input:	None

Output:	None

Return:	Register value

Burst Delay Register (BDELAY)

void Bt848Decoder::SetBurstDelay ( BYTE value )

Purpose:	Set Burst Delay register

Input:	BYTE value - Value to be set to

Output:	None

Return:	None

int Bt848Decoder::GetBurstDelay ( void )

Purpose:	Get Burst Delay register

Input:	None

Output:	None

Return:	Register value

ADC Interface Register (ADC)

void Bt848Decoder::SetAnalogThresholdLow ( bool mode )

Purpose:	Define high/low threshold level below which SYNC signal can be detected

Input:	bool mode - true = low threshold (~75mV), false = high threshold (~125mV)

Output:	None

Return:	None

bool Bt848Decoder::IsAnalogThresholdLow ( void )

Purpose:	Check if high or low threshold level below which SYNC signal can be 			detected

Input:	None

Output:	None

Return:	true = low threshold (~75mV), false = high threshold (~125mV)

void Bt848Decoder::SetAGCFunction ( bool mode )

Purpose:	Enable/Disable AGC function

Input:	bool mode - true = Enable, false = Disable

Output:	None

Return:	None

bool Bt848Decoder::IsAGCFunction ( void )

Purpose:	Check if AGC function is enable or disable

Input:	None

Output:	None

Return:	true = Enable, false = Disable

void Bt848Decoder::PowerDown ( bool mode )

Purpose:	Select normal or shut down clock operation

Input:	bool mode - true = shut down, false = normal operation

Output:	None

Return:	None

bool Bt848Decoder::IsPowerDown ( void )

Purpose:	Check if clock operation has been shut down

Input:	None

Output:	None

Return:	true = shut down, false = normal operation

void Bt848Decoder::SetLumaADC ( bool mode )

Purpose:	Select normal or sleep Y ADC operation

Input:	bool mode - true = normal, false = sleep

Output:	None

Return:	None

bool Bt848Decoder::IsLumaADC ( void )

Purpose:	Check if Y ADC operation is in normal operation or sleeping

Input:	None

Output:	None

Return:	true = normal, false = sleep

void Bt848Decoder::SetChromaADC ( bool mode )

Purpose:	Select normal or sleep C ADC operation

Input:	bool mode - true = normal, false = sleep

Output:	None

Return:	None

bool Bt848Decoder::IsChromaADC ( void )

Purpose:	Check if C ADC operation is in normal operation or sleeping

Input:	None

Output:	None

Return:	true = normal, false = sleep

void Bt848Decoder::SetAdaptiveAGC ( bool mode )

Purpose:	Set adaptive or non-adaptive AGC operation

Input:	bool mode - true = Adaptive, false = Non-adaptive

Output:	None

Return:	None

bool Bt848Decoder::IsAdaptiveAGC ( void )

Purpose:	Check if adaptive or non-adaptive AGC operation is selected

Input:	None

Output:	None

Return:	true = Adaptive, false = Non-adaptive

Software Reset Register (SRESET)

void Bt848Decoder::SoftwareReset ( void )

Purpose:	Perform software reset; all registers set to default values

Input:	None

Output:	None

Return:	None

Test Control Register (TEST)

void Bt848Decoder::AdjustInertialDampener ( bool mode )

Purpose:	For factory diagnostics only

Input:	bool mode - true or false

Output:	None

Return:	None

NOTE:	For factory diagnostics only!!!!!!!