Bt848

Programmer’s Guide

Document Revision 2.25

Driver Version 2.2.223

Brooktreeâ

Brooktree Corporation · 9868 Scranton Road · San Diego, CA 92121-3707 · 619-452-7580

1-800-2-BT-APPS · FAX: 619 452-1249 · Internet apps@brooktree.com

Copyright

Copyright 1996 Brooktree Corporation. All rights reserved.

Brooktree reserves the right to make changes to its products or specifications to improve performance, reliability, or manufacturability. Information furnished by Brooktree Corporation is believed to be accurate and reliable. However, no responsibility is assumed by Brooktree Corporation for its use; nor for any infringement of patents or other rights of third parties which may result from its use. No license is granted by its implication or otherwise under any patent or patent rights of Brooktree Corporation.

Brooktree products are not designed or intended for use in life support appliances, devices, or systems where malfunction of a Brooktree product can reasonably be expected to result in personal injury or death. Brooktree customers using or selling Brooktree products for use in such applications do so at their own risk and agree to fully indemnify Brooktree for any damages resulting from such improper use or sale.

Brooktree is a registered trademark of Brooktree Corporation. Product names or services listed in this publication are for identification purposes only, and may be trademarks or registered trademarks of their respective companies. All other marks mentioned herein are the property of their respective holders.

Specifications are subject to change without notice.

Table Of Contents

Scope	8

Driver Revision History	9

Version 2.2.223 Aug 13 1997	9

Netmeeting and YUV9	9

Registry Key Move	9

Version 2.1.180 July 2 1997	10

Version 2.0.153 Jun 4 1997	10

Version 1.7.3 Apr 29 1997	11

Version 1.7.2 Apr 28 1997	11

Version 1.7.1   Apr 25 1997	11

Version 1.7.0 Apr 22 1997	11

Version 1.6.1: NOT RELEASED	12

Version 1.6.0: NOT RELEASED	12

Version 1.5.0  Apr 7 1997	12

Version 1.4.3  Apr 7 1997	12

Version 1.4.2 : NOT RELEASED	12

Version 1.4.1 Mar 22 1997	12

Version 1.4.0 : NOT RELEASED	13

Version 1.3.0 Feb 22 1997	13

Version 1.2.8 Feb 17 1997	13

Version 1.2.7 : NOT RELEASED	13

Version 1.2.6 Jan 31 1997	13

Documents Referenced	14

Packing List	15

Bt848 Software Features	17

System Requirements	17

Highlights	17

Customer modifiable dialog boxes	18

Translation of Bt848 Error Messages	18

BtGrappler Source Code	18

Microsoft Setup Example Installation Code	18

Tools	18

Bt848 Register Tool	18

Bt848 PCI Configuration Space Tool	18

Bt848 Register Dialog Box	18

MiniRemote control	18

VidCap32	19

I2C Software Tool	19

Direct Draw Capabilities Tool	19

Bt848Dos	19

Notes:	19

Programming Guidelines	20

Turn off VBI streams when switching modes.	20

Capture Rate Issues	21

Closed Caption	22

Performance Issues	22

Closed Caption Callback	22

Defining the Callback	22

Registering the Callback	22

Using the Callback	23

Unregistering the Callback	23

GPIO and I2C Callbacks	24

Direct Draw Surface Issues	25

Surfaces and Bt848	25

Performance Notes	25

Loading a custom Image into the video window	27

Bt848 Software Architecture	28

Overview	28

Figure 1: Bt848 Software ArchitectureRegistry Keys	30

Registry Key Notes	32

AltContrast	32

Bt848 VxD	32

Bt848 Video-for-Windows Capture Driver	32

Bt848 Dialogs	32

Fast & Easy VfW Applications using Microsoft’s AVICap Window Class	33

Overview of the AVICap Window Class	33

Displaying Video	35

Creating a window based on the AVICap Window Class	35

Connecting to the Bt848 VfW capture driver	35

Starting the capture sequence;	35

Stopping the capture sequence	36

Disconnecting from the Bt848 VfW capture driver	36

Controlling the Appearance of the Video	36

Displaying Dialog Boxes for Setting the Video Format	36

Obtaining and Setting the Video Format	36

Previewing Live Video	36

Enabling Overlay of Live Video	37

Bt848 API Functions Description	38

Overview	38

Bt848 I2C Data/Control Register Functions	38

bool I2CIsInitOK( void )	38

ErrorCode I2CInitHWMode( long freq )	38

ErrorCode I2CInitSWMode( long freq )	38

void I2CSetFreq( long freq )	38

int I2CReadDiv( void )	39

ErrorCode I2CHWRead( BYTE address, BYTE *value )	39

ErrorCode I2CHWWrite2( BYTE address, BYTE value1 )	39

ErrorCode I2CHWWrite3( BYTE address, BYTE value1, BYTE value2 )	39

ErrorCode I2CSWStart( void )	39

ErrorCode I2CSWStop( void )	39

ErrorCode I2CSWRead( BYTE * value )	39

ErrorCode I2CSWWrite( BYTE value )	39

ErrorCode I2CSWSendACK( void )	40

ErrorCode I2CSWSendNACK( void )	40

ErrorCode I2CSWSetSCL( Level scl )	40

int I2CSWReadSCL( void )	40

ErrorCode I2CSWSetSDA( Level sda )	40

int I2CSWReadSDA( void )	40

ErrorCode I2CSetSync( State sync )	40

int I2CReadSync( void )	41

int I2CGetLastError( void )	41

BT848 GPIO Functions	41

bool GPIOIsInitOK( void )	41

void SetGPCLKMODE( State s )	41

int GetGPCLKMODE( void )	41

void SetGPIOMODE( GPIOMode mode )	41

int GetGPIOMODE( void )	41

void SetGPWEC( State s )	42

int GetGPWEC( void )	42

void SetGPINTI( State s )	42

int GetGPINTI( void )	42

void SetGPINTC( State s )	42

int GetGPINTC( void )	42

ErrorCode SetGPOEBit( int bit, State s )	42

void SetGPOE( DWORD value )	42

int GetGPOEBit( int bit )	43

DWORD GetGPOE( void )	43

ErrorCode SetGPIEBit( int bit, State s )	43

void SetGPIE( DWORD value )	43

int GetGPIEBit( int bit )	43

DWORD GetGPIE( void )	43

ErrorCode SetGPDATA( GPIOReg * data, int size, int offset )	43

ErrorCode GetGPDATA( GPIOReg * data, int size, int offset )	44

ErrorCode SetGPDATABits( int fromBit, int toBit, DWORD value, int offset )	44

ErrorCode GetGPDATABits( int fromBit, int toBit, DWORD * value, int offset )	44

BT848 Decoder Functions	44

ErrorCode ReadDecoder( WORD regOffset, BYTE * value )	44

ErrorCode WriteDecoder( WORD regOffset, BYTE value )	44

Misc Functions	45

ErrorCode  Bt848SetVideoSource( Connector conn )	45

ErrorCode Bt848GetVideoSource( Connector * conn )	45

ErrorCode Bt848SetBrightness( int brightness )	45

ErrorCode Bt848GetBrightness( int * brightness )	45

ErrorCode Bt848SetContrast( int contrast )	45

ErrorCode Bt848GetContrast( int * contrast )	45

ErrorCode Bt848SetHue( int hue )	45

ErrorCode Bt848GetHue( int * hue )	45

ErrorCode Bt848SetSaturation( int sat )	46

ErrorCode Bt848GetSaturation( int * sat )	46

DWORD Bt848GetStatus( void )	46

ErrorCode Bt848SetSVideo(State s)	46

ErrorCode Bt848GetSVideo( State *s)	46

ErrorCode Bt848SetVideoFormat(VideoFormat vf)	46

ErrorCode Bt848GetVideoFormat ( VideoFormat *vf)	46

void Bt848SetCCCallback( CC_CALLBACK cb )	47

ErrorCode Bt848SetCCStatus( int status )	47

void Bt848SetCCFileName( LPCSTR name )	47

void Bt848SetCCAlert( LPCSTR token )	47

void Bt848GetCCFileName( LPSTR name, int cnt )	47

void Bt848GetCCAlert( LPSTR token, int cnt )	47

int Bt848GetCCStatus( void )	47

ErrorCode Bt848SetImage( LPRECT lpRect, ColFmt aFormat )	48

int Bt848GetImageFormat( ColFmt *pColFrmt )	48

void Bt848GetImageSize( LPRECT lpR )	48

void Bt848Pause( void )	48

void Bt848Continue( void )	48

ErrorCode Bt848ImageLoad ( HINSTANCE hInstance, HWND appMainWnd, LPCSTR szBitmapName )	48

ErrorCode Bt848UnLoadImage ( void )	49

DWORD Bt848GetOverlayFlags()	49

Source Code	50

Dialogs	50

SET PATH=LETTER_DRIVE:\MSVC15\BIN	50

Where ‘LETTER_DRIVE’ is an actual drive letter where you have the compiler installed. Once you have the environment setup, simply build with the provided m_grappler.mak.Source File Descriptions	50

Capture Driver Interface	50

Load	51

Connect	51

Other Linkage	51

Open Video Channels	51

Termination	51

Grappler	51

SET LIB=LETTER_DRIVE:\MSDEV\LIB	51

Source File Descriptions	52

Appendix A: Acronym Definitions	53

Appendix B: WST Interface	54

SECTION 1: VBI data server VxD	54

Overview	54

Bt848 VxD API (applicable to WST interface)	54

Bt848_Get_Version	54

Syntax:	54

Description:	54

Parameters:	54

Returns:	54

Bt848_Open_VBI_Channel	54

Syntax:	54

Description:	54

Parameters:	54

Returns:	55

Bt848_Start_VBI_Capture	55

Syntax:	55

Description:	55

Parameters:	55

Returns:	55

Bt848_Stop_VBI_Capture	55

Syntax:	55

Description:	55

Parameters:	55

Returns:	55

Bt848_Close_VBI_Channel	55

Syntax:	55

Description:	55

Parameters:	55

Returns:	55

Memory Management	56

Callbacks	56

Decoder callback	56

Syntax:	56

Change callback	56

Syntax:	56

Data Structure	56

Session parameters structure	56

Syntax:	56

Description:	56

SECTION 2: VBI decoder VxD (Bt848vbi.vxd)	57

Overview	57

VBI Decoding Implementation	57

Appendix C: Bt848A/Bt849 Features	59

I. Introduction	59

II. Implementation Changes	59

A. Bt848Dlg.dll	59

ErrorCode SetSignalFormat( SignalFormat sFormat )	59

SignalFormat GetSignalFormat( void )	59

ErrorCode SetSyncVideoRef( SyncVideoRef syncRef )	59

SyncVideoRef GetSyncVideoRef( void )	60

ErrorCode SetDecInputClock( DecInputClk iClk )	60

DecInputClk GetDecInputClock( void )	60

ErrorCode SetOutputClock( GPCLKOutputClk oClk )	60

GPCLKOutputClk GetOutputClock( void )	61

ErrorCode SetEnableSyncOutput( State state )	61

State GetEnableSyncOutput( void )	61

ErrorCode SetTimGenByte( TimeGenByte tgb )	61

TimeGenByte GetTimGenByte( void )	61

ErrorCode SetHReset( State state )	61

State GetHReset( void )	61

ErrorCode SetPLL( PLLFreq pll )	62

PLLFreq GetPLL( void )	62

ErrorCode SetVideoInfo( VSIZEPARMS vSize )	62

ErrorCode GetVideoInfo( VSIZEPARMS * vsize )	62

ErrorCode ResetFieldCounter( void )	63

int GetFieldCounter( void )	63

int GetHWRevisionID( void )	63

BOOL IsABt848A( void )	63

III. Windows Registry Key	63

IV. Interfacing with Digital Camera	65

V. Camera Specifications	65

VII. Supporting Tools	66

A. Bt848A Register Tool	66

B. SRAM Load/Dump Utility	66

C. Logitech VideoMan Camera Tool	66

D. Silicon Vision Camera Configuration Tool	66

# Scope

This document serves as a programmer’s guide for the Brooktree Bt848 PCI bus-master video capture integrated circuit.    The audience is assumed to be experienced with programming under Microsoft Windows and Microsoft’s Video for Windows (VfW).    This document explains the major components provided with the Bt848 Software Development Kit (SDK).

The contents of this document has been pulled together from several previous documents, namely Bt848ProgrammerGuide.doc, Bt848SoftwareFeatures.doc, and information found in bt848api.h (previously documented in Bt848dll.doc). New documentation has been added to address the Bt848dlg.dll source code as well as the grappler.dll source code.

# Driver Revision History

## Version 2.2.223 Aug 13 1997

General Notes

##### Netmeeting and YUV9

We have put a great deal of effort into ensuring compatibility with NetMeeting 2.0. A few Bt848 driver bugs were uncovered by testing Netmeeting and have been addressed in this release. There is still one outstanding problem however, and it is not a bug in the Bt848 driver software.    The problem manifests itself as a blank local screen.    This situation occurs when there is no YUV9 codec installed on the system.    Optimally, each system would have Intel’s Indeo codec package installed, but this is not always the case.

The solution to this problem is to ensure that there is a YUV9 codec installed on the system.    Btvvc32.drv contains such a codec and should be installed if there is no other YUV9 codec available. Note that the Bt848 .inf files provided no longer install BtVVC.drv automatically. This is because we do not want to superceed any YUV9 codec present on the system. You should manually install Btvvc32.drv only if there is no other YUV9 codec present (or you need the legacy functionality of Btvvc.drv). An .inf file (Btvvc32.inf) is provided for this purpose.

##### Registry Key Move

As mentioned in a previous release note, we are planning to move our registry keys from “HKEY_CLASSES_ROOT\Bt848”    to “HKEY_LOCAL_MACHINE\SOFTWARE\Brooktree\Bt848”. THIS RELEASE (2.2.223) WILL BE THE LAST RELEASE WITH THE REGISTRY KEYS IN THE OLD LOCATION. SUBSEQUENT RELEASES WILL HAVE THE REGISTRY KEYS IN THE NEW LOCATION.

This release provides the following new features:

- Grappler Source now compiles under Microsoft VC++ v4.2

- Dialogs Source now compiles under Microsoft VC++ v1.5

- Added Teletext Decoder VxD (Bt848VBI.386)

- Distribution of CEBRA teletext applet (interfaces with Bt848VBI.VXD to provide Teletext Display).

- New 32 bit BtVVC32.drv codec. This 32 bit version supercedes previous releases of Btvvc.drv. This release maintains all functionality of the previous version, as well as providing YUV9 codec support.

- Direct Draw 5 support.

This release fixes the following bugs:

- On some slower Pentium systems (below 150 MHz), Intercast was observed to crash. This situation has been corrected.

- An out of bounds PLL register calculation was being allowed.    Range checking was added.

- Netmeeting display was observed to ‘freeze’ during window size update. This has been corrected.

- Netmeeting blank local screen when no YUV9 codec installed. See above discussion under “Netmeeting and YUV9”.

- Some applications were experiencing lockups when invoking capSetVideoFormat. This situation has been corrected.

- Bt848 Interrupt was disabled after Advanced power management Suspend/Resume cycle. This also affected any device sharing the IRQ assigned to Bt848. This has been corrected.

## Version 2.1.180 July 2 1997

This release provides the following new features:

- Intel Intercast support has been enhanced to include support for PAL WST.

This release fixes the following bugs:

- Capture performance during frame drop conditions has been improved.

- Capture time reported for sequences in which no frame drop events occur is now correct.

- Closed captioning now processes carriage returns correctly in roll up mode.

## Version 2.0.153 Jun 4 1997

General Notes

Note that we are at version 2.0.x. We are calling this a major release mainly because of the Bt848A and Bt849 support included in this build. Also note the new version numbering system. The first two digits are the major version number and the minor version number, respectively. The third number is the day of the year the build occurred.

We have run into an issue with the location of our registry keys.    The best location for these keys is “HKEY_LOCAL_MACHINE\SOFTWARE\Brooktree\Bt848”. The new key “GpioInit” has to be in this location since it is read at initialization time. We plan to migrate the existing registry keys from “HKEY_CLASSES_ROOT\Bt848” in the near future. Any new keys we create will be in the new location.

This release provides the following new features:

- Drivers now support Bt848A and Bt849.

- Drivers now setup SIS 5596 and 5511 PCI bridge chips to be compatible with the Bt848.

- Added ability to set Maximum Stretch Threshold (see registry key section below for details).

- Added ability to display an image in the video window (see section titled “Loading a custom Image into the video window” below).    Note that only resource .bmps are supported at this time.    Arbitrary .bmp files will be supported in the next release.

This release fixes the following bugs:

- Closed caption decoder has been modified.    Should provide better weak signal performance.

- Video window was being displayed out side of application under some circumstances in primary surface mode.

- In Win95 Revision 2, the .inf file was not being copied to windows\inf.

- Intercast .inf install failure.

- Various modem induced crashes while running a video app.

- System was locking up when capturing in Vidcap (16 bit).

- Closed caption File Name and Alert Token Text in “Options|Video Display” dialog was corrupted.

- GetOverlayFlags() function was not returning correct status.

- Closed Caption text was being left on video even when Closed Captioning was turned off.

- Registry Keys for GPIO initialization have been moved to: HKEY_LOCAL_MACHINE\SOFTWARE\Brooktree\Bt848\GpioInit

KNOWN BUGS

- Dropped characters while running primary surface mode Closed Captioning.

- Lock up when occluding video window with primary mode closed captioning running.

- GPIO initialization registry keys perform initialization correctly, but when a capture application connects to the driver, the GPIO port is re-initialized (not in accordance with registry keys) This will be addressed in the next release.

- The following sequence produces a flashing toolbar scenario in vidcap32:

- change Regkey "Force Primary" = 1

- Start Vidcap change to 640 x 480, Preview

- Shut down

- Restart Vidcap32

- Select Video Source dialog box

- click OK button

## Version 1.7.3 Apr 29 1997

This release provides the following new features:

- Added CC primary surface control registry keys.

KNOWN BUGS

- If video window is moved while Closed Captioning is enabled, text may appear at the top left of the screen until the end of the line is processed. The text will remain until a clear screen code is processed. Workaround: If this happens, turn off Closed Captioning, then turn it on again. This problem will be addressed in the next release.

## Version 1.7.2 Apr 28 1997

This release provides the following new features:

- Added primary surface mode Closed Captioning

## Version 1.7.1      Apr 25 1997

This release provides the following new features:

- Added WST (World Standard Teletext) Interface for Bt849.

- Added support for GPIO register Initialization registry keys

- Added 32 bit GetSVideo & SetSVideo calls.

This release fixes the following bugs:

- GPIO Interrupt enable bit is now left alone by the video engine.

- Fixed bug where capture rates of .001 and .002 produces a “Can not capture” error message, but capture took place anyway.

## Version 1.7.0 Apr 22 1997

This release provides the following new features:

- Enhanced Closed Caption decoder performance

- .Inf Support for Bt849.

This release fixes the following bugs:

- Magenta bar bug in VidCap16

## Version 1.6.1: NOT RELEASED

## Version 1.6.0: NOT RELEASED

## Version 1.5.0    Apr 7 1997

This release provides the following new feature:

- Improvements to low-signal level locking performance.

## Version 1.4.3    Apr 7 1997

This release fixes the following bugs:

- Fixed GPF during capture if the target disk is full.

- Closed captioning decoding support fixed (was broken in 1.4.1)

KNOWN LIMITATIONS

( we do not consider these as "bugs", but you should be aware of them)

- VfW preview mode performance can be severely degraded when the CC decoder is on.    We suggest that you disable CC in preview mode.

## Version 1.4.2 : NOT RELEASED

## Version 1.4.1 Mar 22 1997

This release fixes the following bugs:

- PAL max vertical size 284.    Previous version used 2 fields for PAL sizes > 284 lines.    Now the driver uses 1 field for V <= 288; 2 fields for V > 288.

- Stale data in capture files.    In long capture sequences it was observed that old frames & fields would appear: ie. The driver was repeating some old frames.

- Corrupted/partial data in capture files.    The driver was placing "garbage" or partial frames/fields into capture files.

- Capture Frame errors:    further improvements in frame-rate accuracy have been made.

- Interlace error: in 2 field mode (ie. V > 240 for NTSC, V > 288 for PAL) sometimes the driver would reverse the order of the fields.

The following methodology was used to verify these fixes:

- Captured AVI file's at 5-25 FPS (30 for NTSC) from SMPTE time coded source.

- Single step through frames; verify that there are no frames out of sequence by visually examining the timecode information.

- Visually verify that there are no corrupt frames.

- Calculate FPS base on time coded information in AVI file, verify correct FPS

- The following shows the video formats, capture formats and sizes verified:

|  | NTSC | PAL |
| --- | --- | --- |
| YUV9 | 640x480 | 768x576 |
|  | 320x240 | 384x288 |
|  | 160x120 | 192x144 |
| RGB24 | 640x480 | 768x576 |
|  | 320x240 | 384x288 |
|  | 160x120 | 192x144 |

KNOWN BUGS

- We have seen the driver GPF during capture if the target disk is full.

- Closed captioning decoding support is disabled.

## Version 1.4.0 : NOT RELEASED

## Version 1.3.0 Feb 22 1997

This release fixes the following bugs:

- Pal Capture Problems: numerous performance related issue involving PAL capture were addressed.    You should thoroughly retest your PAL capture    features.

- Wrong version info was being embedded in AVI files.    The Version info embedded in avi file was always 1.0.0.0.    Now the AVI file version info while using will correctly be the same as the capture driver version ( eg. 1.3.0 in this case)

- S-Video input comes up monchrome.    The S-Video input was starting up in B&W mode.

This release provides the following new feature:

- We have added an alternate contrast adjustment algorithm. This attempts to keep the brightness more constant as the contrast is varied.    This is a work-around to a HW limitation in the Bt848 decoder section.    This is enabled by setting the AltContrast key to 1.

## Version 1.2.8 Feb 17 1997

- Adds support for S3 Virge problem identified in Release 1.2.6

## Version 1.2.7 : NOT RELEASED

## Version 1.2.6 Jan 31 1997

This release provides the following major features:

- Direct Draw 3 support: DD2 and 1 will no longer be supported.

- Closed Captioning:a) dramatically improved decoder performance; b) dramatically improved on-screen display performance in DDraw overlay mode ( still no support for primary surface mode ).

- Capture improvements: capture performance has been greatly enhanced. We believe that we have identified and cured the root cause of numerous problems associated with capture.

- The SetSVideo functions now work correctly

Known problems

- S3 Virge DX2: there is a problem with the DX2 version of the S3 Virge that causes the palette to become corrupted.    S3 is currently working on a fix.    This is not a Bt848 problem !    There is no problem with the plain Virge chips.

# Documents Referenced

- Bt848 Data Sheet

- I2C-Bus Reference Guide

- DecoderPermissions.txt

# Packing List

The following table summarizes the files distributed as the Bt848 SDK:

| Directory or File | Description |
| --- | --- |
| Root Directory of Distribution Tree |  |
| Bt848.inf | Sample .inf file for Bt848 driver installation. |
| Bt848ICAST.inf | Sample .inf for Intercast installation |
| readme.txt | Release Notes |
| Subdirectory Docs |  |
| Bt848ProgrammerGuide.doc | This Document |
| L848001A.PDF | Bt848 Data Sheet. Use Adobe Acrobat to view/print. |
| DecoderPermissions.txt | Complete list of decoder register read/write permissions. |
| Subdirectory Drv |  |
| Bt848.drv | Capture Driver |
| Bt848.VXD | VxD Portion of Capture Driver. |
| Bt848DDI.dll | Direct Draw Interface DLL. |
| Bt848Dlg.dll | Capture Driver Dialogs DLL. |
| Bt848_16.dll | I2C and GPIO thunk layer DLL. |
| Bt848_32.dll | I2C and GPIO services DLL |
| Bt84xcc.dll | Closed caption decoder DLL. |
| BtVVC.drv | VfW CODEC. Supports Y41P FOURCC. Used primarily for BtV. |
| grappler.dll | Grappler board specific interface to Bt848_32.DLL Implements Intercast TV control API |
| Subdirectory GrapplerHW |  |
| PCB\GERBER\ | Directory contains GERBER Files for 848EVM (Grappler) Hardware Design. |
| PCB\GERBER\\_contents.TXT | See this file for complete contents of this dir. |
| PCB\PS\ | PostScript version of PCB Design. |
| PCB\PS \_contents.TXT | See this file for complete contents of this dir. |
| SCH\ | Contains Orcad and PostScript versions of the schematic. Also contains Bill Of Materials. |
| SCH\_contents.TXT | See this file for complete contents of this dir. |
| Subdirectory Source |  |
| dialogs\ | Contains full source code for Bt848dlg.dll (the Capture driver dialogs DLL) |
| grappler\ | Contains full source code for grappler.dll. |
| lib\bt848api.h | Header to be included when interfacing to Bt848_xx.dll. |
| lib\Bt848_16.lib | Link to this lib to use Bt848_16.DLL |
| lib\bt848_32.def | bt848_32.DLL export definitions |
| lib\Bt848_32.lib | Link to this lib to use Bt848_32.DLL |
| lib\Bt848_16.dll | I2C and GPIO thunk layer DLL. |
| lib\Bt848_32.dll | I2C and GPIO services DLL |
| lib\i2cerr.h | Error codes for Bt848_xx.DLL API Functions |
| lib\MS848_32.exp | Export file generated by Microsoft LIB command |
| lib\MS848_32.lib | Use to link with Microsoft Visual C++ 4.x Applications |
| Subdirectory Tools | See ‘Tools’ section below for more complete descriptions. |
| Bt848RegTool.exe | Bt848 Register tool. |
| I2csw16.exe | Software Control I2C test program (16 bit) |
| I2csw32.exe | Software Control I2C test program (32 bit) |
| miniRemote.exe | Mini Remote Control (Source Included) |
| PCICONFG.exe | PCI Configuration Space Tool |
| RegDlg.exe | Single Register Tool |
| VIDCAP32.exe | Video Capture Application. |
| DD_Caps.exe | Direct Draw Capabilities Application. |
| Bt848Dos.exe | DOS Bt848 sample application. |
| Subdirectory Tools\Source |  |
| CC_Test\ | Source code for Closed Caption Callback Test application. |
| Ring0CallbackTest\ | Source Code for simple Ring 0 Callback test application and VxD. |

# Bt848 Software Features

This section provides an overview of the capabilities of the Bt848 drivers, sample source code, documentation and tools distributed in the Bt848 Software Development Kit.

## System Requirements

- Direct Draw 3.0 Provider

- Graphics Card with the following:

- Linear Frame Buffer

- Hardware Cursor

## Highlights

Video for Windows Capture Driver

- All requirements of Microsoft Video for Windows Capture driver are met.

- Provides Microsoft-defined interface for creating video capture and display applications.

- Capture formats supported:

- RGB 32, 24, 15, 8

- YUV12, 9

- YUY2

- BtYUV

All Video Standards Supported

- NTSC

- PAL (all versions that are supported by the hardware)

- SECAM

Complex Clipping

- Support for a minimum of 8 simultaneous clipping regions

Fully-Variable Down Scaling

- Scaling range determined by video standard and hardware capabilities

Decoder control functions

- Allows control of decoder parameters - e.g. brightness, contrast, input selection, etc.

Closed Caption Support

- Closed Caption Decoder

- On Screen Display Integrated with Capture Driver

- Closed Caption Alarm

- Capture of Closed Caption Text to disk

Intel Intercast Ready

- All required interfaces between Intel Intercast software and Bt848 drivers are implemented.

I2C support

I2C (Inter-Integrated Circuit) is a multi-master serial bus used for communications between cooperating ICs. The Bt848 is an I2C master, and can program other devices via this bus. See Brooktree’s I2C Reference Guide for details.

- Software I2C master allows infinite length transactions without stops or re-starts.

- Full software support of the hardware’s I2C master.

GPIO support

- Interface functions to GPIO port are provided..

## Customer modifiable dialog boxes

- Full standard C source code to Bt848 Video for Windows dialog boxes. Allows modification of user interface to suit customer’s application.

- Provided with makefiles for Borland C++ 5.0 and Microsoft Visual C++.

## Translation of Bt848 Error Messages

There may be situations where the various error messages displayed by the Bt848 capture driver need to be modified by the customer (translation to another language is the most common example). This can be accomplished by using a standard resource editor such as Borland’s Resource Workshop.

Follow these steps to edit the resource strings (We highly recommend backing up any files that you edit!):

- Open Borland's Resource Workshop (all directions below assume RW used ).

- Open a project ( File|Open Project ). Select DRV in the File Type entry box. Select Bt848.drv

- All resources in the driver are displayed.

- Select a resource to be modified and change it.

- Save the project ( File| Save project ).

Note: There is a limit on the length of the strings in the resources ( 254 chars).

## BtGrappler Source Code

- Full standard C++ source code to BtGrappler DLL gives concrete example of using Bt848 in a typical TV Tuner/video capture design.

- Includes standard C++ source code for controlling Temic tuners.

- Provided with makefiles for Borland C++ 5.0 and Microsoft Visual C++.

## Microsoft Setup Example Installation Code

- Demonstrates typical Microsoft installation process for Bt848 software.

## Tools

### Bt848 Register Tool

- Displays all Bt848 local registers simultaneously.

- Allow modification of local registers.

- Allows local register set to by stored and loaded from a file.

### Bt848 PCI Configuration Space Tool

- Displays all Bt848 configuration registers simultaneously.

- Allows modification of Bt848 configuration registers.

### Bt848 Register Dialog Box

- Displays a single Bt848 local register.

- Allow modification of local registers.

### MiniRemote control

- Bare-bones remote control.

- Make calls only into grappler.dll.

- Works with 848 EVM Board (BtGrappler) hardware (no audio support except mute/unmute).

### VidCap32

- Microsoft’s textbook sample capture application from the Video for Windows SDK.

### I2C Software Tool

- Infinite loop to change channels using software I2C for BtGrappler hardware.

### Direct Draw Capabilities Tool

- Determines hardware and Direct Draw emulation capabilities.

### Bt848Dos

This directory contains a complete DOS based Bt848 example application. Also included is the DJCPP compiler which the example is built with.

## Notes:

- For each I2C hardware function there exists a corresponding software API.

- Source code: The Bt848dlg.dll, BtGrappler, Temic Tuner , MiniRemote and Bt848.inf installation source code is provided as an example only.    THIS CODE IS NOT SUPPORTED BY BROOKTREE CORPORATION. IT IS PROVIDED "AS IS" BECAUSE WE BELIEVE IT MAY BE USEFUL TO YOU.

- Tools: THESE TOOLS ARE NOT SUPPORTED BY BROOKTREE CORPORATION. THEIR PROPER OPERATION IS NOT GUARANTEED. THEY ARE PROVIDED "AS IS" BECAUSE WE BELIEVE THEY MAY BE USEFUL TO YOU.

# Programming Guidelines

This section is included to bring certain salient points to programmer’s attention. These are things that are not inherently obvious, but have been found to make sense in the context of the Bt848/9 and driver software.

## Turn off VBI streams when switching modes.

When Closed Captioning is on (closed captioning is encoded in the VBI) is it wise to turn it off before changing the state of the video in any way.    One example is changing the channel. Although the driver does time out closed captioning and erase the display, if closed captioning is not turned off before changing channels, errant text may appear on the screen.

In fact, we recommend turning off any VBI stream before any kind of mode change.

# Capture Rate Issues

The capture driver has a maximum frame per second (fps) rate of 30 for NTSC and 25 for PAL. Note that in non-interlaced resolutions, only the odd field is processed. For the driver to accomplish greater than the maximum frame rate capture accuracy, both fields would have to be processed in these non-interlaced resolutions. In the interest of performance, the current design of the driver does not process both fields at non-interlaced resolutions.

Note however that Video for Windows will attempt to keep track of video streams at greater than max fps. VfW does this by inserting old frames in place of the unavailable frame. VfW assumes the frame has been dropped in this case, but in reality the driver simply cannot provide the frame since it doesn’t really exist. Therefore, the capture driver cannot guarantee accuracy at frame rates above the maximums of 25 fps for PAL and 30 fps for NTSC.

In other words, even though it is possible to specify a value greater than the maximum fps via the capture video dialog, accuracy beyond these maximum values is not guaranteed.

# Closed Caption

## Performance Issues

If the Bt848 is displaying video on the primary surface, system performance may be adversely affected while closed caption display is enabled. Performance difficulties usually manifest themselves as dropped closed caption characters (usually dropped pairs of characters). The reason for the performance trouble is the amount of work required to set up the clipping regions for the closed caption text. This problem is more evident if a “paint-on” closed caption stream is being processed. The problem is especially evident in screen sizes over 240 pixels high.

Two registry keys are provided to disable closed caption display in primary surface mode. The keys are optional and intended to be used in the event of the above described performance problems. The keys are:

- KillPrimaryCC	Disables all primary surface mode closed captioning if set to “1”.

- KillPrimaryCCFull	Disables closed captioning for screen sizes above 240 pixels primary surface if set to “1”. Note that this key has no effect if KillPrimaryCC is set to “1”.

## Closed Caption Callback

The capture driver provides access to decoded closed caption bytes through a user defined callback. Once the callback is registered with the capture driver, the capture driver calls it at the regular Windows timer frequency (200 mSec) as long as there are decoded bytes to return. When called, the callback is passed a pointer to the decoded CC characters and the length of the buffer in bytes.

The closed caption decoder retrieves characters from line 21 only. Parity is checked on each decoded byte. If the parity check fails, bit 7 (mask = 0x80) of the character is set. Note that all characters encoded (including pad bytes (0x00) and control codes) will show up in the buffer passed to the callback. The buffer, therefore, cannot be treated as a null terminated string.

A test program for the closed caption callback feature has been provided in the TOOLS\SOURCE\CC_TEST directory of the distribution.

### Defining the Callback

The callback must reside in a 16 bit DLL or have _loadds attribute. The example code in this section is assumed not to be in a DLL.

void CALLBACK _loadds _export CallbackFunc( LPCSTR cc, int ccLen )

{

}

### Registering the Callback

First we must define a few things, specifically the CC_CALLBACK function pointer type, A pointer type for the registration function, and finally the actual pointer to the registration function:

typedef void ( CALLBACK *CC_CALLBACK)( LPCSTR, int );

typedef void ( WINAPI * SetCCCallback_t ) ( CC_CALLBACK );

SetCCCallback_t pSetCCCallback;

The registration function resides in Bt848dlg.dll. We must now obtain a pointer to the registration function. We begin by loading the DLL, then we use GetProcAddress() to get the function pointer.

// load DLL which contains the install callback function

hDlgDLL = LoadLibrary( "Bt848dlg.dll" );

if ( hDlgDLL < HINSTANCE_ERROR ){

MessageBox( "Missing file: Bt848dlg.dll\n", "Error", MB_ICONSTOP );

return;

}

// look for the install callback function

pSetCCCallback = (SetCCCallback_t)GetProcAddress( (HMODULE)hDlgDLL,

"SetCCCallback" );

if ( pSetCCCallback == NULL ){

MessageBox( "Cannot find \"SetCCCallback\" function.\n", "Error", MB_ICONSTOP );

return;

}

Now that we have a valid pointer to the registration function, we can finally register our callback:

static FARPROC pThunk;    // ptr to thunk to the callback function

pThunk = MakeProcInstance( (FARPROC)CallbackFunc, *GetModule() );

if ( pSetCCCallback && *pSetCCCallback )

(*pSetCCCallback)( (CC_CALLBACK) pThunk );

### Using the Callback

When the callback has been successfully registered, it will be called periodically when closed caption data has been decoded. Note that the data returned is “raw”, meaning that the byte stream contains control codes specific to closed captioning. The callback routine will need to parse the incoming data for control codes and take appropriate action. Display characters should be buffered for display.

### Unregistering the Callback

It is important to unregister the callback when it is no longer needed (i.e. application termination).

The following code unregisters the callback:

if ( pSetCCCallback && *pSetCCCallback )

(*pSetCCCallback)( NULL );

FreeProcInstance( pThunk );

# GPIO and I2C Callbacks

The Bt848 Virtual Device Driver (Bt848.vxd) provides callback capability for devices attached to the Bt848 I2C and/or GPIO ports. The callbacks are invoked in response to I2CDONE and GPINT interrupts. No parameters are passed to the callback routines.

The VXD provides the following services to register a user’s ring 0 callback routine:

VxD_Service( BT848_Set_GPIO_Callback )

VxD_Service( BT848_Set_I2C_Callback )

The registration functions are prototyped as follows:

static void    _cdecl BT848_Set_GPIO_Callback( DWORD cb );

static void    _cdecl BT848_Set_I2C_Callback( DWORD cb );

Note: parameter ‘cb’ is the 32 bit linear address of the callback function.

The following is an example of a callback routine:

void GPIO_Callback()

{

DEBUGTRACE( "GPIO callback" );

}

The appropriate interrupt is enabled when a callback is registered, and masked when the callback is unregistered (by passing NULL to the registration function). For example:

To register a GPIO callback and enable the interrupt:

BT848_Set_GPIO_Callback( DWORD( GPIO_Callback ) );

To unregister the callback and mask the interrupt:

BT848_Set_GPIO_Callback( NULL );

Note that the Bt848 interrupt status bit for the GPIO or I2C is cleared by the VxD immediately before the callback is invoked. Since the VxD clears the appropriate bits before a callback is invoked, your application should never rely on the state of the GPIO or I2C interrupt status bits.

For a complete example program see the Tools\Source\Ring0CallbackTest directory in the SDK distribution. View the readme.txt file for instructions on how to build and execute the test code.

# Direct Draw Surface Issues

## Surfaces and Bt848

Bt848 drivers have capability to display live video over PCI bus directly into the graphics frame buffer. In order to achieve this, Bt848 needs to know an address in the frame buffer. Microsoft's DirectDraw interface (DDI) is the tool to obtain such an address.

DDI has a notion of a surface. It encapsulates attributes and operations that can be performed by a drawing applications on memory (i.e. surface). A surface in this context is a pixel data buffer and set of functions that operate on that data ( i.e. blitting ).

Two surfaces that Bt848 utilizes are primary surface and overlay surface. Primary surface is the one that is directly visible ( screen ). Overlay surface is exactly what its name says it is. Data from an overlay surface can be overlaid on top of primary surface. Primary surface is usually in one of the RGB formats; overlay surface usually is in one of the YUV formats. Overlay hardware support of a graphics chip performs color space translation when displaying overlay data. Overlaying is based on the principle of color keying. A region of primary surface is painted with so called key color ( Bt848 uses magenta ). Overlay hardware compares color of the primary surface with the key color and when they match data from overlay surface's buffer is displayed; if colors do not match graphics are displayed. Overlay surface uses position data in addition to the key color ( overlay hardware must know where video should be displayed ).

Bt848 drivers use the following scheme when working with DirectDraw. First an attempt is made to create the overlay surface. If successful, it is used. Bt848 transfers data into overlay surface ( somewhere in the off-screen memory ). This is the preferred way. No clipping issues are present, as key color takes care of this. If a drop-down menu obscures the video window, then the key color will be replaced with the menu colors    and overlay hardware will fail the key color comparison. One big advantage of overlay is that screen can be in 8bpp mode ( so user can have highest screen resolution ), but no video image degradation happens as hardware uses RGB24 when it does color keying. Another advantage of using overlay surface is that upscaling can be performed. That's how video can be stretched to cover the entire screen. Some graphics controllers have difficulties implementing overlay. S3 Virge exhibits a bandwidth problem, ATI's 264 VT has limited overlay width. Tseng    does not support color key natively.

If overlay surface creation fails, or is unsupported by the graphics controller, primary surface is used to display video. In this mode Bt848 transfers video pixels directly into visible portion of frame memory. No color key is used and clipping information is required. DirectDraw provides an API to obtain clip list information for a given window. When such clip list changes, Bt848's RISC program is updated to accommodate it. To do so RISC program is paused, modified and restarted. Another disadvantage of using primary surface is that in 256 color mode image quality suffers.

## Performance Notes

For the same video window size bandwidth can be significantly lower for overlay surface than primary. The reason is overlay surface can scale up. A 320x240 window, in primary surface 320x240xsurface_bpp is transferred for every field; for overlay surface 160x120x16 ( a 16 bpp YUV format is used most likely ) is transferred for every field. Primary surface requires 4 times more bandwidth than overlay ( provided prim surface is in 16bpp mode). Currently, the Virge chip uses this method to support overlay. There can be some scaling artifacts because of this..

Some overlay graphics controllers    may not have implemented the PCI bus correctly and may in fact slow it down. . So, primary surface can yield higher performance.

With many drivers    there is no difference in performance between overlay and primary when screen is in 16 bpp mode; and primary is using ½ of overlay bandwidth when screen is in 8bpp mode.

In general, it is only with some hardware analyzers that one can see difference between    overlay vs. primary surface support.

# Loading a custom Image into the video window

Two functions have been added to BT848_32.DLL:

ErrorCode Bt848ImageLoad ( HINSTANCE hInstance, HWND appMainWnd, LPCSTR szBitmapName )

ErrorCode Bt848UnLoadImage ( void )

You will need to load the bt848_32.dll module into the current app, then use GetProcAddress to obtain pointers for these functions, i.e.,

typedef DWORD (WINAPI *DDILoadImage)(HINSTANCE hInst, HWND hWnd, LPCSTR szBitmap);

typedef DWORD (WINAPI *DDIUnLoadImage)( void );

// pointers to functions of the above type

DDILoadImage pLoadImage=NULL;

DDIUnLoadImage pUnLoadImage=NULL;

HANDLE hDLL = LoadLibrary ( "Bt848_32" );

if ( !hDLL )

return Error;

pLoadImage      = ( DDILoadImage )    GetProcAddress( hDLL, "Bt848ImageLoad" );

pUnLoadImage = ( DDIUnLoadImage )GetProcAddress( hDLL, "Bt848UnLoadImage" );

To load an image:    (for now, we only support .bmp format)

If your bitmap is a resource:

if    (pLoadImage( hInstance, ghMainWnd, MAKEINTRESOURCE (RES_NAME)) != Success )

{

//    Error

}

If your bitmap is not a resource, then you need to specify the complete path    (from root) to the bitmap.

char szFilename [] = "c:\picture\sample.bmp";

if    (pLoadImage( hInstance, ghMainWnd, szFilename ) != Success )

{

//    Error

}

To remove the bitmap from the display:

if ( pUnLoadImage( ) != Success )

{

// error    (Never happens)

}

# Bt848 Software Architecture

## Overview

The Bt848 SDK consists of the following major components:

Bt848.vxd	= the Bt848 virtual device driver;

Bt848.drv 	= the Bt848 VfW capture driver;

Bt848Ddi.dll	= the Bt848 Direct Draw Interface;

Bt848Dlg.dll	= the dialogs called by Bt848.drv;

Bt848_32.dll	= 32-bit Bt848 GPIO and I2C support;

Bt848_16.dll	= 16-bit Bt848 GPIO and I2C support;

Bt848.inf	= Microsoft Windows setup file.

The following diagram illustrates the interaction of these components with each other and with other Windows components, as well as interaction with an end user capture application.

### Figure 1: Bt848 Software ArchitectureRegistry Keys

Several Win95/WinNT registry keys are maintained by the Bt848 software. The following table describes each key.

| HKEY_LOCAL_MACHINE\ SOFTWARE\Brooktree\Bt848 |  |
| --- | --- |
| GpioInit | Used to initialize the GPIO interface during driver init. Has the following optional string values (this key is also optional): GPCLKMODE GPIOMODE GPWEC GPINTI GPINTC GPOE GPIE GPDATA See Bt848 Data Sheet for descriptions of these registers. |
| HKEY_CLASSES_ROOT\Bt848 |  |
| AltContrast | subkey to enable brightness compensation during contrast adjustments. ; 0 to disable compensation ; 1 to enable compensation, |
| Brightness | Value from Video Format Dialog |
| Color Format | 0 = RGB32 1 = RGB24 2 = RGB16 3 = RGB15 4 = YUY2 5 = BTYUV 6 = Y8 7 = RGB8 8 = PL_422 9 = PL_411 10 = YUV9 11 = YUV12 12 = VBI 13 = UYVY 14 = RAW 15 = I420 |
| Connector | 1 = Svideo 2 = Tuner 3 = Composite |
| Contrast | Value from Video Format Dialog |
| ErrorDialog | Enables/Disables Bt848.drv error dialogs. |
| Force Primary | subkey to force using primary surface ; 0 to try to create overlay surface first (default) ; 1 to force using primary surface |
| Hue | Value from Video Format Dialog |
| KillPrimaryCC | Disables all primary surface mode closed captioning if set to “1”. |
| KillPrimaryCCFull | Disables closed captioning for screen sizes above 240 pixels primary surface if set to “1”. Note that this key has no effect if KillPrimaryCC is set to “1”. |
| Maximum Stretch Threshold | The software checks for the presence of this registry key and if not present, the key is created and set to the default value of 3500 (which represents a maximum scale-down of 3.5:1). If the key is present, the key value is used to determine the maximum scale down attempted when attempting to create an overlay surface. |
| NumXTAL | subkey for the number of crystals installed on your board (1 or 2) |
| PreviewOnly | Forces Bt848.drv to use VfW preview only (ie. no overlay) |
| Rect Height | Height of video |
| Rect Left | Leftmost position of video; always 0 |
| Rect Top | Topmost position of video; always 0 |
| Rect Width | Width of video |
| Saturation | Value from Video Format Dialog |
| Surface | Read Only (set by the driver) 0 = Overlay Surface 1 = Primary Surface |
| Svideo | Not currently used |
| VBI_HDelay | Offset used to affect the starting point of VBI waveform capture |
| VBI_PacketSize | No longer used. |
| VideoDisplayDialog | Enables/Disables display of Bt848.drv video display dialog box. Causes driver to return DV_ERR_NOTSUPPORTED in response to the WM_CAP_DLG_VIDEODISPLAY message. |
| Video Format | NTSC, PAL |
| Virge | subkey for Virge chip ; 0 for no Virge chip (default) ; 1 for Virge chip |
| XT0 | subkey for the video standard associated with crystal ; attached to XT0 (NTSC,PAL,NONE) |
| XT1 | subkey for the video standard associated with crystal ; attached to XT1 (NTSC,PAL,NONE) |

The following keys are set by Bt848 drivers, but used by Intercast:

| HKEY_CLASSES_ROOT\Intercast.Display |  |
| --- | --- |
| DisplayBitMask | Indicates what video sizes are available. This is a 4 bit mask: Bit 0: means QCIF Bit 1 - CIF Bit 2 -Full size Bit 3 - full screen. When set to 1 given size is supported, set to 0 means size not supported. Usually, only bit 3 can be set to 0. |
| Surface | Indicates whether primary or overlay surface is used. 1 means primary, 0 means overlay |

#### Registry Key Notes

##### AltContrast

If AltContrast key is set to 1, changes to the Bt848 brightness register are minimized as the contrast is adjusted.

The formulae are:

To keep brightness near constant (i.e. not affected by changing contrast)

set brightness to B/(C/C0)

To adjust the contrast level such that it is at the middle of black and white:

set brightness to (B * C0 + 64 * (C0 - C))/C

Where:

B is value of brightness before adjustment

C is contrast value

C0 is nominal contrast value (0xD8)

### Bt848 VxD

Bt848.VxD is a Brooktree proprietary component that provides a system interface to the Bt848.    Some of the functions that the VxD provides are DMA control, video clip-list management and Windows 95 Configuration Manager services.    The VxD is distributed as a binary only.    There are no programmer modifiable components or API.

### Bt848 Video-for-Windows Capture Driver

Bt848.drv is a Brooktree proprietary component that implements a complete Microsoft Video for Windows capture driver.    Please refer to the appropriate Microsoft documentation for further information.    The capture driver    is distributed as a binary only.

### Bt848 Dialogs

Bt848Dlg.dll contains code and resources for controlling the Bt848’s video capture and display parameters.    This DLL is called by the capture driver when a VfW application requests a user interface function such as adjusting the video contrast. The DLL supports the following messages to display VfW Dialogs:

- WM_CAP_DLG_VIDEODISPLAY

- WM_CAP_DLG_VIDEOFORMAT

- WM_CAP_DLG_VIDEOSOURCE

Complete source for Bt848dlg.dll is provided in the SDK. See the section entitled Dialogs on page 47 for more complete information.

## Fast & Easy VfW Applications using Microsoft’s AVICap Window Class

### Overview of the AVICap Window Class

The Microsoft VfW Development Kit provides all the libraries, tools and sample code that you will need to produce a VfW capture application.

The term capture is used to denote both saving a video stream to a file and displaying the video in a window on the monitor.

Microsoft’s AVICap window class encapsulates all of the details of streaming video capture to AVI files. You do not need to be involved with the details of the AVI file format, video buffer management, or the low-level access of video device drivers. The AVICap window class provides similar support for audio.

You can add video capture capabilities to an application with only the following lines of code:

hwndC = capCreateCaptureWindow ("My Capture Window", WS_CHILD | WS_VISIBLE , 0, 0, 160, 120, hwndParent, nID);

SendMessage (hwndC, WM_CAP_DRIVER_CONNECT, 0 /*wIndex*/, 0L);

SendMessage (hwndC, WM_CAP_SEQUENCE, 0, 0L);

Using other AVICap messages, your application can similarly control most other aspects of the video capture process.

You are encouraged to use the macros provided by Microsoft in MMSYSTEM.H and VFW.H.    These macros isolate your application from the SendMessage interface, making the transition from Win16 to Win32® easier:

hwndC = capCreateCaptureWindow ("My Own Capture Window", WS_CHILD | WS_VISIBLE ,0, 0, 160, 120, hwndParent, nID);

capDriverConnect (hwndC, 0);

capCaptureSequence (hwndC);

AVICap windows are similar to standard Windows controls. The client application creates the capture window, and then sends control messages to it with SendMessage.

The following illustration shows the architecture of the AVICap window class.

Figure 2: AVICap Window Class Architecture

### Displaying Video

You need to perform the following steps to capture or display a video stream:

- Create a window based on the AVICap Window Class;

- Connect to the Bt848 VfW capture driver;

- Start the capture sequence;

- Stop the capture sequence;

- Disconnect from the Bt848 VfW capture driver.

#### Creating a window based on the AVICap Window Class

This function creates an AVICap capture window.

HWND FAR PASCAL _loadds capCreateCaptureWindow(LPCSTR lpszWindowName, DWORD dwStyle, int x, int y, int nWidth, int nHeight, HWND hWnd, int nID)

Returns a handle to the window if the function was successful, otherwise it returns NULL.

lpszWindowName - Specifies a zero-terminated string containing the name used for the capture window.

dwStyle Specifies the window styles used for the AVICap capture window. Window styles are described with the CreateWindow function.

x - specifies the x-coordinate of the upper-left corner of the AVICap capture window.

y - Specifies the y-coordinate of the upper-left corner of the AVICap capture window.

nWidth - Specifies the width of the AVICap capture window.

nHeight - Specifies the height of the AVICap capture window.

hWnd - Specifies the handle to the parent window.

nID - Specifies the window ID.

#### Connecting to the Bt848 VfW capture driver

This function establishes the connection to the capture driver:

BOOL capDriverConnect(HWND hwndCapture, WPARAM ndxOfDriver) ;

Returns TRUE if connection is successful, else FALSE.

hwndCapture - Handle to capture window.

ndxOfDriver      - Specifies the index of the video-capture device driver corresponding to its SYSTEM.INI entry. Video-capture device drivers are identified by the key MSVIDEO in the [drivers] section of the SYSTEM.INI file. Device indexes correspond to the indexes following the MSVIDEO key. These indexes can be noncontiguous and range between 0 and 9.

#### Starting the capture sequence;

One of the key features of Bt848 is the ability to support overlay in software using Microsoft’s DirectX Direct Draw Interface (DDI).    This feature enables your application to display real-time video images in systems that have graphics cards with DDI support.

The following code fragment show how to enable overlay on the Bt848:

CAPDRIVERCAPS capDriverCaps;

capDriverGetCaps (hwndCapture, &capDriverCaps, sizeof (CAPDRIVERCAPS));

if (capDriverCaps.fHasOverlay)

capOverlay (hwndCapture, TRUE);

#### Stopping the capture sequence

This function ends the capture sequence:

BOOL capStop(HWND hwndCapture) ;

Returns TRUE if connection is successful, else FALSE.

hwndCapture - Handle to capture window.

#### Disconnecting from the Bt848 VfW capture driver

This function breaks the connection to the capture driver:

BOOL capDriverDisconnect(HWND hwndCapture) ;

Returns TRUE if connection is successful, else FALSE.

hwndCapture - Handle to capture window.

### Controlling the Appearance of the Video

#### Displaying Dialog Boxes for Setting the Video Format

Each video-capture device driver can provide up to three different dialog boxes to control aspects of the video digitization and capture process. (In the VidCap application, these three dialog boxes are invoked by three entries in the Options menu.) The video source dialog box controls selection of video input channels and controls parameters such as hue, brightness, and contrast. The video format dialog box controls selection of the dimensions of the captured video and bit-depth of the captured video. The video display dialog box controls the appearance of the video on the monitor during capture, but it does not actually affect the video during digitization (that is, it controls post-capture effects only). The contents of these dialog boxes are defined by the video-capture driver (In the Bt848.drv implementation, the video display dialog box controls Closed Caption capture only).

One additional dialog box lets the user set the post-capture video compression attributes. The contents of this dialog box depend on the compression format selected.

If supported by a video-capture device driver, your application can use the following messages to display the dialog boxes. The WM_CAP_DLG_VIDEODISPLAY message displays a dialog box used to control the video output. The WM_CAP_DLG_VIDEOFORMAT message displays a dialog box used to select the video format. The WM_CAP_DLG_VIDEOSOURCE message displays a dialog box used to control the video source.

### Obtaining and Setting the Video Format

An application can use the WM_CAP_GET_VIDEOFORMAT message to obtain the video format data structure or its size, and it can use WM_CAP_SET_VIDEOFORMAT to set the format for capturing video.

### Previewing Live Video

In preview mode, frames are transferred from the capture hardware to system memory and then displayed in the capture window using GDI functions. The WM_CAP_SET_PREVIEW and WM_CAP_SET_PREVIEWRATE messages control the preview operation. The WM_CAP_SET_PREVIEW message enables or disables preview mode. The WM_CAP_SET_PREVIEWRATE message sets the rate at which frames are displayed in preview mode.

Client applications might decrease the preview rate when the parent loses focus, and increase the preview rate when the parent gains focus. This improves general system responsiveness, because the preview operation is processor intensive.

Enabling video preview automatically disables video overlay.

### Enabling Overlay of Live Video

The Bt848 supports the display of live video using overlay. This hardware function displays the video on the VGA screen independent of the central processor. Applications can use the WM_CAP_SET_OVERLAY message to enable and disable video overlay. Enabling video overlay automatically disables video preview.

# Bt848 API Functions Description

## Overview

Bt848_32.dll provides developers with a set of application programming interfaces (APIs) necessary to create applications to communicate with the Bt848. Sixteen bit applications should interface with Bt848_16.dll, the thunk layer. This document describes the API functions developed to manipulate the I2C Data/Control and the General Purpose I/O (GPIO) related registers of the Bt848. There are four GPIO related registers that can be modified and retrieved through invoking the functions listed later in the documentation. The related GPIO registers are GPIO and DMA Control, GPIO Output Enable Control, GPIO Registered Input Control, and GPIO Data I/O registers.

Descriptions for the I2C Data/Control register functions are listed in the next section whereas descriptions for the GPIO and related registers are listed in later section. The last section provides information of the APIs developed to read and write the decoder register.

## Bt848 I2C Data/Control Register Functions

This section of the documentation describes the functions developed to manipulate I2C Data/Control register of the Bt848.

### bool I2CIsInitOK( void )

Purpose:	Check if I2C is initialized successfully

Input:	None

Output:	None

Return:	true or false

Note:	Before calling other I2C functions, application should call this function and check for return value is true to ensure initialization is successful.

### ErrorCode I2CInitHWMode( long freq )

Purpose:	Initialize I2C for hardware control of SCL and SDA

Input:	long freq - frequency (hz) to run SCL at

Output:	None

Return:	Success (zero) or Fail (non-zero)

Note:	After calling I2CIsInitOK(), application should call this function and check for return value is Success before starting hardware communication.

### ErrorCode I2CInitSWMode( long freq )

Purpose:	Initialize I2C for software control of SCL and SDA

Input:	long freq - frequency (Hz) to run SCL at

Output:	None

Return:	Success (zero) or Fail (non-zero)

Note:	After calling I2CIsInitOK(), application should call this function and check for return value is Success before starting software communication.

### void I2CSetFreq( long freq )

Purpose:	Set frequency for SCL

Input:	long freq - frequency (Hz) to run SCL at (137.5khz to 2.0625Mhz) PCI frequency: 33Mhz: SCL = (412.50Khz to 33.81Khz) 25Mhz: SCL = (312.50Khz to 25.61Khz)

Output:	None

Return:	None

### int I2CReadDiv( void )

Purpose:	Obtain value of the programmable divider

Input:	None

Output:	None

Return:	Value of the programmable divider

### ErrorCode I2CHWRead( BYTE address, BYTE *value )

Purpose:	Perform a hardware read from the I2C

Input:	BYTE address		- address to be read from

Output:	BYTE *value		- retrieved value

Return:	Success (zero) or Fail (non-zero)

### ErrorCode I2CHWWrite2( BYTE address, BYTE value1 )

Purpose:	Perform a hardware write of two bytes to the I2C

Input:	BYTE address		- address to be written to

BYTE value1		- value of 2nd byte to be written

Output:	None

Return:	Success (zero) or Fail (non-zero)

### ErrorCode I2CHWWrite3( BYTE address, BYTE value1, BYTE value2 )

Purpose:	Perform a hardware write of three bytes to the I2C

Input:	BYTE address		- address to be written to

BYTE value1		- value of 2nd byte to be written

BYTE value2		- value of 3rd byte to be written

Output:	None

Return:	Success (zero) or Fail (non-zero)

### ErrorCode I2CSWStart( void )

Purpose:	Initiate I2C transaction in software mode

Input:	None

Output:	None

Return:	Success (zero) or Fail (non-zero)

### ErrorCode I2CSWStop( void )

Purpose:	Terminate I2C transaction in software mode

Input:	None

Output:	None

Return:	Success (zero) or Fail (non-zero)

### ErrorCode I2CSWRead( BYTE * value )

Purpose:	Read a byte from I2C in software mode

Input:	None

Output:	BYTE * value 		- value read from I2C

Return:	Success (zero) or Fail (non-zero)

### ErrorCode I2CSWWrite( BYTE value )

Purpose:	Write a byte to I2C in software mode

Input:	BYTE value 		- value to be written to I2C

Output:	None

Return:	Success (zero) or Fail (non-zero)

### ErrorCode I2CSWSendACK( void )

Purpose:	Send ACK to slave

Input:	None

Output:	None

Return:	Success (zero) or Fail (non-zero)

### ErrorCode I2CSWSendNACK( void )

Purpose:	Send NACK to slave

Input:	None

Output:	None

Return:	Success (zero) or Fail (non-zero)

### ErrorCode I2CSWSetSCL( Level scl )

Purpose:	Set software SCL value

Input:	Level scl 		- Hi releases the SCL output, Low forces the 				    SCL output low

Output:	None

Return:	Success (zero) or Fail (non-zero)

### int I2CSWReadSCL( void )

Purpose:	Read software SCL value

Input:	None

Output:	None

Return:	SCL value 		- Hi releases the SCL output, Low forces the 				    SCL output low.

### ErrorCode I2CSWSetSDA( Level sda )

Purpose:	Set software SDA value

Input:	Level sda 		- Hi releases the SDA output, Low forces the 				    SDA output low.

Output:	None

Return:	Success (zero) or Fail (non-zero)

### int I2CSWReadSDA( void )

Purpose:	Read software SDA value

Input:	None

Output:	None

Return:	SDA value 		- Hi releases the SDA output, Low forces the 				    SDA output low.

### ErrorCode I2CSetSync( State sync )

Purpose:	Set I2C sync value

Input:	State sync		- On allow slave to insert wait states, Off    				    disallow slave to insert wait states.

Output:	None

Return:	Success (zero) or Fail (non-zero)

### int I2CReadSync( void )

Purpose:	Read I2C sync value

Input:	None

Output:	None

Return:	sync value:		- On: allow slave to insert wait states, Off: 				    disallow slave to insert wait states.

### int I2CGetLastError( void )

Purpose:	Obtain last I2C error number

Input:	None

Output:	None

Return:	Last I2C error number

## BT848 GPIO Functions

This section of the documentation describes the functions developed to manipulate the General Purpose I/O (GPIO) registers of the Bt848. There are four GPIO related registers that can be modified and retrieved through invoking the functions listed below. The related GPIO registers are GPIO and DMA Control, GPIO Output Enable Control, GPIO Registered Input Control, and GPIO Data I/O registers. Refer to the Bt848 Data Sheet for detailed descriptions of these registers.

### bool GPIOIsInitOK( void )

Purpose:	Check if GPIO is initialized successfully

Input:	None

Output:	None

Return:	true or false

Note:	Before calling other GPIO functions, application should call this function and check for return value is true to ensure initialization is successful.

### void SetGPCLKMODE( State s )

Purpose:	Set or clear GPCLKMODE

Input:	State s 			- On to set; Off to clear

Output:	None

Return:	None

### int GetGPCLKMODE( void )

Purpose:	Get value of GPCLKMODE

Input:	None

Output:	None

Return:	int 			- On (1) or Off (0)

### void SetGPIOMODE( GPIOMode mode )

Purpose:	Set GPIO mode

Input:	GPIOMode mode 	- GPIO_NORMAL, GPIO_SPI_OUTPUT, 				    GPIO_SPI_INPUT,    GPIO_DEBUG_TEST

Output:	None

Return:	None

### int GetGPIOMODE( void )

Purpose:	Get GPIO mode

Input:	None

Output:	None

Return:	int			- GPIO_NORMAL, GPIO_SPI_OUTPUT, 				    GPIO_SPI_INPUT, GPIO_DEBUG_TEST

### void SetGPWEC( State s )

Purpose:	Set or clear GPWEC

Input:	State s 			- On to set; Off to clear

Output:	None

Return:	None

### int GetGPWEC( void )

Purpose:	Get value of GPWEC

Input:	None

Output:	None

Return:	int 			- On (1) or Off (0)

### void SetGPINTI( State s )

Purpose:	Set or clear GPINTI

Input:	State s		 	- On to set; Off to clear

Output:	None

Return:	None

### int GetGPINTI( void )

Purpose:	Get value of GPINTI

Input:	None

Output:	None

Return:	int 			- On (1) or Off (0)

### void SetGPINTC( State s )

Purpose:	Set or clear GPINTC

Input:	State s		 	- On to set; Off to clear

Output:	None

Return:	None

### int GetGPINTC( void )

Purpose:	Get value of GPINTC

Input:	None

Output:	None

Return:	int 			- On (1) or Off (0)

### ErrorCode SetGPOEBit( int bit, State s )

Purpose:	Set or clear a bit in GPOE

Input:	int bit			- bit to be set or clear

State s			- On to set; Off to clear

Output:	None

Return:	ErrorCode 		- Success (0) or Fail (non-zero)

### void SetGPOE( DWORD value )

Purpose:	Set GPOE register content

Input:	DWORD value 		- value to be set to

Output:	None

Return:	None

### int GetGPOEBit( int bit )

Purpose:	Get value of a bit in GPOE

Input:	int bit 			- bit to get value from

Output:	None

Return:	int 			- On (1), Off (0), or -1 for parameter error

### DWORD GetGPOE( void )

Purpose:	Get register content of GPOE

Input:	None

Output:	None

Return:	Content of GPOE (DWORD)

### ErrorCode SetGPIEBit( int bit, State s )

Purpose:	Set or clear a bit in GPIE

Input:	int bit			- bit to be set or clear

State s			- On to set; Off to clear

Output:	None

Return:	ErrorCode - Success (0) or Fail (non-zero)

### void SetGPIE( DWORD value )

Purpose:	Set GPIE register content

Input:	DWORD value	 	- value to be set to

Output:	None

Return:	None

### int GetGPIEBit( int bit )

Purpose:	Get value of a bit in GPIE

Input:	int bit 			- bit to get value from

Output:	None

Return:	int 			- On (1), Off (0), or -1 for parameter error

### DWORD GetGPIE( void )

Purpose:	Get register content of GPIE

Input:	None

Output:	None

Return:	Content of GPIE (DWORD)

### ErrorCode SetGPDATA( GPIOReg * data, int size, int offset )

Purpose:	Set GPDATA register content

Input:	GPIOReg * data	- pointer to data buffer to be copied from

int size			- number of locations to copy; max is 64

int offset		- number of locations to skip (default = 0)

Output:	None

Return:	ErrorCode - Success (0) or Fail (non-zero)

### ErrorCode GetGPDATA( GPIOReg * data, int size, int offset )

Purpose:	Get GPDATA register content

Input:	GPIOReg * data 	- pointer to data buffer to be copied to

int size			- number of locations to copy; max is 64

int offset		- number of locations to skip (default = 0)

Output:	None

Return:	ErrorCode - Success (0) or Fail (non-zero) if error in parameter

Note:	This function can only retrieve the last value written to GPDATA

### ErrorCode SetGPDATABits( int fromBit, int toBit, DWORD value, int offset )

Purpose:	Set GPDATA contents with specified range of bits

Input:	int fromBit		- starting bit

int toBit			- ending bit

DWORD value		- value to set to

int offset		- number of locations to skip (default = 0)

Output:	None

Return:	ErrorCode - Success (0) or Fail (non-zero) if error in parameter

Comment:	By specifying a range of bits, it allows a portion of the DATA I/O register to be modified. For example, SetGPDATA( 8, 10, 5, 0 ) would set bits [10:8] of the first GPDATA mapped location to value 101(binary)

### ErrorCode GetGPDATABits( int fromBit, int toBit, DWORD * value, int offset )

Purpose:	Get GPDATA contents with specified range of bits

Input:	int fromBit		- starting bit

int toBit			- ending bit

int offset		- how many locations to skip (default = 0)

Output:	DWORD * value	- value retrieved

Return:	ErrorCode - Success (0) or Fail (non-zero) if error in parameter

Comment:	By specifying a range of bits, it allows a portion of the GDATA I/O register to be retrieved. For example, GetGPDATA( 8, 10, &data, 0 ) would retrieve bits [10:8] of the first GPDATA mapped location to *data

Note:	This function can only retrieve the last value written to GPDATA

## BT848 Decoder Functions

There are two functions available to perform read and write operations of the decoder registers. Read function allows retrieval of any decoder register’s content. However not all decoder register contents are permitted to be modified with the write function. For a complete list of the read/write permissions of these registers, refer to the file “DecoderPermissions.txt”.

### ErrorCode ReadDecoder( WORD regOffset, BYTE * value )

Purpose:	Read a decoder register's content

Input:	WORD regOffset	- offset of the register to retrieve (must be in 				    DWORD boundary)

Output:	BYTE * value		- byte read from decoder

Return:	Success or Fail

### ErrorCode WriteDecoder( WORD regOffset, BYTE value )

Purpose:	Modify a decoder register's content

Input:	WORD regOffset	- offset of the register to modify (must be in 				    DWORD boundary)

BYTE value		- value to set to

Output:	None

Return:	Success or Fail

## Misc Functions

### ErrorCode    Bt848SetVideoSource( Connector conn )

Purpose:	Request VxD to set video source

Input:	Connector conn - ConCamera, ConSVideo, ConTuner, ConComposite

Output:	None

Return:	Success or Fail

### ErrorCode Bt848GetVideoSource( Connector * conn )

Purpose:	Ask VxD what video source has set to

Input: 	None

Output:    	Connector * conn - ConCamera, ConSVideo, ConTuner, ConComposite

Return:    	Success or Fail

### ErrorCode Bt848SetBrightness( int brightness )

Purpose: 	Request VxD to set video brightness

Input:      	int brightness - brightness value

Output:    	None

Return:    	Success or Fail

### ErrorCode Bt848GetBrightness( int * brightness )

Purpose: 	Ask VxD what brightness value has set to

Input:      	None

Output:    	int * brightness - brightness value

Return:    	Success or Fail

### ErrorCode Bt848SetContrast( int contrast )

Purpose: 	Request VxD to set video contrast

Input:      	int contrast - contrast value

Output: 	None

Return:    	Success or Fail

### ErrorCode Bt848GetContrast( int * contrast )

Purpose: 	Ask VxD what contrast value has set to

Input:      	None

Output:    	int * contrast - contrast value

Return:    	Success or Fail

### ErrorCode Bt848SetHue( int hue )

Purpose: 	Request VxD to set video hue

Input:      	int hue - hue value

Output:    	None

Return:    	Success or Fail

### ErrorCode Bt848GetHue( int * hue )

Purpose: 	Ask VxD what hue value has set to

Input:      	None

Output:    	int * hue - hue value

Return:    	Success or Fail

### ErrorCode Bt848SetSaturation( int sat )

Purpose: 	Request VxD to set video saturation

Input:      	int sat - saturation value

Output:    	None

Return:    	Success or Fail

### ErrorCode Bt848GetSaturation( int * sat )

Purpose: 	Ask VxD what saturation value has set to

Input:      	None

Output:    	int * sat - saturation value

Return:    	Success or Fail

### DWORD Bt848GetStatus( void )

Purpose: 	Obtain STATUS register content

Input:      	None

Output:    	None

Return:    	status register content

### ErrorCode Bt848SetSVideo(State s)

Purpose: 	Request VxD to set status of SVideo

Input:      	State s - {Off, On}

Output:    	None

Return:    	Success or Fail

### ErrorCode Bt848GetSVideo( State *s)

Purpose: 	Request tp VxD to get status of SVideo

Input:      	State *s - A pointer to variable of type State.

Output:    	State *s - {Off,On}

Return:    	Success or Fail

### ErrorCode Bt848SetVideoFormat(VideoFormat vf)

Purpose: 	Request VxD to set status of SVideo

Input:      	VideoFormat vf - { VFormat_AutoDetect, VFormat_NTSC, VFormat_NTSC_J,    VFormat_PAL_BDGHI, VFormat_PAL_M,            VFormat_PAL_N, VFormat_SECAM,    VFormat_PAL_NC }

Output:    None

Return:    Success or Fail

### ErrorCode Bt848GetVideoFormat ( VideoFormat *vf)

Purpose: 	Request tp VxD to get status of SVideo

Input:	VideoFormat vf - A pointer to variable of type VideoFormat.

Output:	VideoFormat *vf - { VFormat_AutoDetect, VFormat_NTSC, VFormat_NTSC_J,          VFormat_PAL_BDGHI, VFormat_PAL_M,            VFormat_PAL_N, VFormat_SECAM,            VFormat_PAL_NC }

Return:	Success or Fail

### void Bt848SetCCCallback( CC_CALLBACK cb )

Purpose:	Register a callback for external decode of CC VBI data.

Input:      	CC_CALLBACK cb - pointer to a function of type CC_CALLBACK (typedef void ( CALLBACK *CC_CALLBACK)( LPBYTE buf, int cnt);)

Output:    	None

Return:    	None

### ErrorCode Bt848SetCCStatus( int status )

Purpose:	Turns CC VBI processing off and on.

Input:      	int status - Only bits 15 and 0 are used. All others must be set to zero. Set bit 15 to enable CC VBI Processing. If bit 0 is 0 then CC processing occurs on channel 2, if bit 0 is set to 1 then CC processing occurs on channel 1.

Output:    	None.

Return:    	Success or Fail

### void Bt848SetCCFileName( LPCSTR name )

Purpose:	Sets the CC text capture filename

Input:      	LPCSTR name - Fully qualified file name

Output:    	None.

Return:    	Success or Fail

### void Bt848SetCCAlert( LPCSTR token )

Purpose:	Sets a text string that, when found in the CC stream, causes a dialog to appear (indicating that the string was found).

Input:      	LPCSTR token - Null terminated string intended to trigger response.

Output:    	None

Return:    	None

### void Bt848GetCCFileName( LPSTR name, int cnt )

Purpose:	Obtain the current CC text capture filename.

Input:      	LPSTR name 	- Pointer to buffer to receive name	,

int cnt		- Size of buffer

Output:    	‘name’ buffer receives current CC text capture filename

Return:    	None

### void Bt848GetCCAlert( LPSTR token, int cnt )

Purpose:	Obtains the current CC Alert token. (See Bt848SetCCAlert)

Input:      	LPSTR token 	- Pointer to buffer to receive token

int cnt		- Size of buffer

Output: 	‘token buffer receives current CC Alert token

Return:    	None

### int Bt848GetCCStatus( void )

Purpose:	Obtain the current operational ststus of the CC decoder

Input:      	None

Output:    	None

Return: 	int - Only bits 15 and 0 are used. Bit 15 set indicates CC VBI processing is enabled. If bit 0 is 0 then CC processing is set for channel 2, if bit 0 is set to 1 then CC processing is set for channel 1.

### ErrorCode Bt848SetImage( LPRECT lpRect, ColFmt aFormat )

Purpose:	Sets the capture driver image size and color format.

Input: 	LPRECT lpRect - pointer to bounding rectangle

ColFmt aFormat - color format : typedef enum { CF_BelowRange = -1, CF_RGB32, CF_RGB24, CF_RGB16, CF_RGB15, CF_YUY2,      CF_BTYUV, CF_Y8, CF_RGB8, CF_PL_422, CF_PL_411, CF_YUV9, CF_YUV12, CF_VBI,      CF_UYVY, CF_RAW = 0x0E, CF_I420, CF_AboveRange} ColFmt;

Output:    	None

Return:    	Success or Fail

### int Bt848GetImageFormat( ColFmt *pColFrmt )

Purpose:	Obtain current capture driver image color format.

Input:      	ColFmt *pColFrmt - pointer to a ColFmt type : typedef enum { CF_BelowRange = -1, CF_RGB32, CF_RGB24, CF_RGB16, CF_RGB15, CF_YUY2,      CF_BTYUV, CF_Y8, CF_RGB8, CF_PL_422, CF_PL_411, CF_YUV9, CF_YUV12, CF_VBI,      CF_UYVY, CF_RAW = 0x0E, CF_I420, CF_AboveRange} ColFmt

Output: 	Location pointed to by pColFmt contains the current color format.

Return: 	int - Number of bpp for the current format

### void Bt848GetImageSize( LPRECT lpR )

Purpose: 	Obtain current capture driver image size

Input:      	LPRECT lpR - pointer to RECT structure to be filled

Output:    	RECT structure filled with bounding rectangle of current capture driver image.

Return:    	None

### void Bt848Pause( void )

Purpose:	Pauses the Bt848 Risc Engine

Input:      	None.

Output:    	None.

Return:    	None.

### void Bt848Continue( void )

Purpose:	Restarts Bt848 Risc Engine

Input:      	None

Output:    	None

Return:    	None

### ErrorCode Bt848ImageLoad ( HINSTANCE hInstance, HWND appMainWnd, LPCSTR szBitmapName )

Purpose:	Load a bitmap into the video window

Input:	HINSTANCE hInstance	    Instance handle of calling application.

HWND appMainWnd	    Window handle of calling application.

LPCSTR szBitmapName    Full path to bitmap (.bmp).

Output:	None.

Return:	Success or Fail

### ErrorCode Bt848UnLoadImage ( void )

Purpose:	Removes image loaded with Bt848ImageLoad()

Input:	None.

Output:	None.

Return:	Success or Fail

### DWORD Bt848GetOverlayFlags()

Purpose:	Returns the type of direct draw surface currently being used by the driver for “live” video.    Note that the value returned is undefined if preview mode is active.

Input:	None.

Output:	None.

Return:	USE_OVERLAY (0x1) if overlay surface, USE_PRIMARY (0x2) if primary surface.    These constants are defined in ddiflags.h.

# Source Code

## Dialogs

This is the full source code to Bt848dlg.dll. This source is provided to enable OEMs to modify the dialogs to suit their needs. This customization could include enhancing functionality or merely changing the appearance of the dialogs.

Dialogs.dll is now buildable under Microsoft Visual C++ 1.5 as well as Borland C++. If you are going to compile under MSVC, you’ll need to set up the environment as follows:

SET INCLUDE=LETTER_DRIVE:\MSVC15\INCLUDE

SET LIB=LETTER_DRIVE:\MSVC15\LIB

###### SET PATH=LETTER_DRIVE:\MSVC15\BIN

### Where ‘LETTER_DRIVE’ is an actual drive letter where you have the compiler installed. Once you have the environment setup, simply build with the provided m_grappler.mak.Source File Descriptions

- api.h	- Interface to I2C and GPIO services.

- audio.h	- Defines audio source types.

- bt848api.h	- Exported functions in Bt848_32.dll.

- Bt848Dlg.lib	- Current lib file for linking this dll.

- Bt848Dlg.mak	- Borland C++ 5.0 Makefile.

- cap_api.cpp	- Interface to capture driver.

- colfrmat.h	- Color Format typedef.

- config.h	- Video Configuration Dialog defines.

- common.h	- Typedefs common to more than one target.

- dcdrvals.h	- Video adjustment constants.

- ddiflags.h	- Direct Draw Interface flags.

- def__.h	- Some defines to be used in other header files.

- dialogs.cpp	- Contains main dialog proc.

- dialogs.def	- Linker definition file.

- dialogs.h	- Main header file; defines dialog indices in .rc.

- dialogs.rc	- Resource file (#includes the .dlg files).

- exviddlg.h	- Defines some dialog constants.

- fourcc.h	- Defines the fourcc values supported by the Bt848 drivers.

- gpiotype.h	- Defines GPIO related registers and GPIO mode.

- m_bt848_16.def	- Linker definition file used during Microsoft compile.

- m_Bt848Dlg.mak	- Microsoft Visual C++1.5 Makefile.

- retcode.h	- Defines error codes.

- viddefs.h	- Defines many enumerated types used in source code.

- viddispl.dlg	- Overlay Configuration dialog definition.

- vidfmt.dlg	- Video Format dialog definition.

- vidsrc.dlg	- Video Source dialog definition.

### Capture Driver Interface

This section describes the interaction between Bt848dlg.dll and the capture driver when the capture driver is loaded.

#### Load

Immediately after loading ( inside LibMain(), actually ), the capture driver attempts to establish a connection with the video dialogs. To do so it performs LoadLibrary() call to bring Bt848dlg.dll into memory. If the call is successful, the capture driver attempts to obtain 4 entry points inside Bt848dlg.dll:

- Connect

- VideoDialog

- Initialize

- Disconnect

#### Connect

After the entry points are obtained, the Connect() API entry point is called. This is the best place for Bt848dlg.dll to obtain all the entry points ( API ) from the capture driver ( cap_api.cpp    file contains all the needed code to do so ).

#### Other Linkage

Bt848dlg.dll in turn implicitly links to the Bt848_16.dll which contains 16 bit thunks for the I2C and GPIO functions. These functions can be called after DLL is loaded.

#### Open Video Channels

Usually, when application connects to a capture driver, three video channels are opened in the driver:

- External video in

- External video out

- Video in

If the PreviewOnly registry key is set to 1, the capture driver calls the Initialize() API entry point after two or more channels are opened. If the PreviewOnly registry key is set to 0, the Initialize() API entry point is called after all three channels are opened.    At that point Bt848dlg.dll can call any of the exported functions in the capture driver.

#### Termination

When a VfW application terminates, all video channels are closed. At some point the driver receives the DRV_FREE message. In response to this message, the Disconnect() API entry point is called. After that, FreeLibrary() frees the Bt848dlg.dll from memory.

## Grappler

BtGrappler is a sample implementation of the Intercast TV control API and tuner control. Source code for the implementation of Grappler.dll to control BtGrappler hardware is provided as an example of how to implement the communication.

Grappler.dll is now buildable under Microsoft Visual C++ 4.x as well as Borland C++. If you are going to compile under MSVC, you’ll need to set up the environment as follows:

SET INCLUDE=LETTER_DRIVE:\MSDEV\INCLUDE

###### SET LIB=LETTER_DRIVE:\MSDEV\LIB

SET PATH=LETTER_DRIVE:\MSDEV\BIN

Where ‘LETTER_DRIVE’ is an actual drive letter where you have the compiler installed. Once you have the environment setup, simply build with the provided m_grappler.mak.

### Source File Descriptions

- audio.h	- Defines audio source types.

- bt848api.h	- Exported function in Bt848_32.dll.

- bt848fcn.h	- Exported function in Bt848_32.dll.

- common.h	- Typedefs common to more than one target.

- def__.h	- Some defines to be used in other header files.

- gpiotype.h	- Defines GPIO types.

- grappler.def	- def file for grappler.dll.

- grappler.mak	- Makefile for grappler.dll with Borland C++ 5.0.

- grappler.rc	- Resource file.

- grappler.rh	- Resource header file.

- interfac.cpp	- Source code for exported functions in grappler.dll.

- m_bt848_32.def	- Linker definition file used for Microsoft compile.

- m_grappler.def	- def file for grappler.dl (Used for Microsoft compile).

- m_grappler.mak	- Makefile for grappler.dll with Microsoft Visual C++ 4.x.

- retcode.h	- Defines ErrorCode enumerated type.

- tuner.cpp	- Source code to control the Temic tuner.

- tuner.h	- Defines tuner structures and Tuner class.

- tvctl.h	- Defines TV parameters, source, error values and exported functions.

- tverr.h	- Lists all control error values.

- viddefs.h	- Defines many enumerated types used in source code.

# Appendix A: Acronym Definitions

| ACK | Acknowledge |
| --- | --- |
| API | Application Programming Interface |
| Bt | Brooktree |
| BtYUV | Brooktree enhanced YUV format. |
| CC | Closed Captioning |
| DDI | Direct Draw Interface |
| DLL | Dynamic Link Library |
| DMA | Direct Memory Access |
| GPIO | General Purpose I/O |
| GPOE | GPIO Output Enable |
| I2C | Inter-Integrated Circuit Interface. |
| NACK | Negative Acknowledge |
| NTSC | National Television Standards Committee: North America’s TV transmission standard. |
| PAL | Phase Alternation Line: Europe’s TV transmission standard. |
| PCI | Peripheral Components Interface |
| RGB | Red, Green, Blue : color space format used for computer displays. |
| SCL | Serial Clock on the I2C Bus |
| SDA | Serial Data on the I2C Bus |
| SDK | Software Development Kit |
| SECAM | Another TV format similar to PAL |
| VfW | Video for Windows |
| VxD | Virtual Device Driver |
| YUV | Color space format. Y is the luminance component, U and V are the color components. Variants include YUV9 (useful for Indeo) and YUV12 (useful for MPEG, video conferencing). |

# Appendix B: WST Interface

## SECTION 1: VBI data server VxD

### Overview

This section describes the interface between the Bt848 VxD ( the VBI data server) and a VBI decoder VxD ( the VBI data client).

A VBI client VxD needs to perform the following steps:

1) Obtain Bt848 version.

2) Secure access to the VBI resource.

3) Start VBI capture.

4) Process the data in the decoder's callback function.

5) Stop VBI capture.

6) Release VBI resource.

Only one VBI client may be connected to the Bt848 VxD at any one time. This includes the Bt848 internal Closed Caption decoder.

### Bt848 VxD API (applicable to WST interface)

#### Bt848_Get_Version

##### Syntax:

VxDCall( Bt848_Get_Version );

##### Description:

This service is used to test for the presence of the Bt848 VxD and to obtain a version number.

##### Parameters:

None

##### Returns:

On error CF is set. On success CF is cleared, AH contains major version number, AL contains minor version number.

#### Bt848_Open_VBI_Channel

##### Syntax:

VxDCall( Bt848_Open_VBI_Channel );

##### Description:

This service is used to secure the VBI resources (including data buffers).

##### Parameters:

EDX - address of    the session parameters structure (see section 6).

##### Returns:

On error CF is set. On success CF is cleared, EAX contains a handle to be used in all other functions. Failure can occur due to the fact the VBI resource is used by someone else or memory could not be allocated for VBI data buffers.

#### Bt848_Start_VBI_Capture

##### Syntax:

VxDCall( Bt848_Start_VBI_Capture );

##### Description:

This service is used to start VBI capture.

##### Parameters:

EDX - handle returned by the Bt848_Open_VBI_Channel

##### Returns:

On error CF is set. On success CF is cleared. Error can occur if someone else has locked the VBI resource ( handle in EDX is invalid ).

#### Bt848_Stop_VBI_Capture

##### Syntax:

VxDCall( Bt848_Stop_VBI_Capture );

##### Description:

This service is used to stop VBI capture.

##### Parameters:

EDX contains the handle returned by the Bt848_Open_VBI_Channel

##### Returns:

On error CF is set. On success CF is cleared. Error can occur if someone else has locked the VBI resource ( handle in EDX is invalid )

#### Bt848_Close_VBI_Channel

##### Syntax:

VxDCall( Bt848_Close_VBI_Channel );

##### Description:

This service is used release the VBI resources (including data buffers).

##### Parameters:

EDX contains the handle returned by the Bt848_Open_VBI_Channel

##### Returns:

On error CF is set. On success CF is cleared. Error can occur if someone else has locked the VBI resource ( handle in EDX is invalid )

### Memory Management

The Bt848 VxD creates and manages memory for the VBI data.    The data stream is double-buffered.    The VBI Client VxD must process (or copy) the data within 2 video frame times.    Data in the buffers is overwritten as new data becomes available from the input video signal.    The Bt848 VxD releases the buffer memory when the VBI VxD calls Bt848_Close_VBI_Channel.

### Callbacks

#### Decoder callback

##### Syntax:

EBX - contains the buffer address

EDX - contains the tag passed in the session parameters structure

ECX - indicates even/odd field. Zero means odd. Non-zero means even

#### Change callback

##### Syntax:

ECX - contains the flag that indicates what is changing. 0 means channel change; 1 means video format change.

EDX - depending on the value in EAX contains either channel number or video format value. Video format values are the same as used by the capture driver and decoder.

### Data Structure

#### Session parameters structure

##### Syntax:

typedef struct {

DWORD dwOversampling;

DWORD dwPitch;

DWORD dwLines;

DWORD dwDecoderCallback;

DWORD dwChangeCallback;

DWORD dwTag;

}

##### Description:

(output = data provided by VBI server Vxd to client)

(input    = data provided by VBI client Vxd to server)

dwOversampling (output) Specifies the sample rate for VBI digitization relative to the video input frequency. The DWORD is structured as follows bits 31-24 contain integer part;    bits 23-0 contain fractional part.

dwPitch (output) Contains the number of bytes from the first byte of a line to the first byte    of the next line dwLines output Number of VBI lines being captured.

dwDecoderCallback (input) Address of a function that will be called to decode a field of VBI data

dwChangeCallback (input) Address of a function to receive notifications about channel changes and    video format changes

dwTag (input) Arbitrary data to be passed to the decoder callback

## SECTION 2: VBI decoder VxD (Bt848vbi.vxd)

### Overview

This section describes the interface between the Bt848VBI VxD (the WST decoder server) and a WST client. Note that the VxD ID for Bt848VBI.VxD is 3C18h.

#### VBI Decoding Implementation

The VBI decoding algorithm is implemented in a 32-bit virtualdevice driver, BT848VBI.VXD. It uses the data call-back function of the BT848.VXD to supply the raw samples of the digitised VBI

lines. A 32-bit VxD API is provided for interfacing to a VxD at a higher level. For efficiency, and to avoid buffering, the BT848VBI.VXD uses a call-back function (similar to the one used by BT848.VXD) to forward the packets to the next level of the protocol.

The VxD API is described below. All functions set the carry flag (CF) on error, leaving an error code in EAX. On success, CF will be cleared.

Bt848VBI_GetVersion()

Purpose:	Called to test for presence of the VxD and to get the version number.

Parameters:	None

Returns:	On success, AX contains the version number (AH major, AL

minor).

Bt848VBI_Open()

Purpose:	Called to establish a connection to a client.

Parameters:	EDX is a pointer to a parameter structure:

struct

{

UN32 EvenVBIMask; // All lines: 0x1FFFF

UN32 OddVBIMask; // All lines: 0x3FFFF

UN32 PacketCallback;

UN32 ChangeCallback;

};

EvenVBIMask tells on which VBI lines in the even field VBI decoding should be attempted. Bit 0 corresponds to VBI line 6, etc. If a bit is cleared, VBI decoding will not be attempted on the corresponding VBI line (saves CPU power).

OddVBIMask tells on which VBI lines in the even field VBI decoding should be attempted. Bit 0 corresponds to VBI line 318, etc. If a bit is cleared, VBI decoding will not be attempted on the corresponding VBI line (saves CPU power).

PacketCallback is a pointer to a function that will be called when a teletext packet has been decoded.

ChangeCallback is a pointer to a function that will be called when the video mode/source has changed.

Returns:	On success, EAX contains a non-zero handle to be used in all other functions. Only one client can get access to the VxD at a time.

Bt848VBI_Close()

Purpose:	Used to terminate a connection to a client.

Parameters:	EBX contains handle for the connection.

Returns:	Nothing.

Bt848VBI_Start()

Purpose:	Used to (re-)start the VBI decoding process. Once started, the PacketCallback function is called as teletext packets are decoded.

Parameters:	EBX contains handle for the connection.

Returns:	Nothing.

Bt848VBI_Stop()

Purpose:	Used to stop the VBI decoding process. While stopped, the PacketCallback function will not be called.

Parameters:	EBX contains handle for the connection.

Returns:	Nothing.

PacketCallback()

Purpose:	Called by the VBI decoder when a teletext packet has been decoded. The VBI decoder performs no buffering of data, so the call-back function must copy the received packet to a buffer as needed. The call- back function must not perform time-consuming tasks, because it must expect to be called 18 times every 20ms (worst case).

Parameters:	EAX contains the number of the VBI line the packet was decoded from (6-22 (odd),    318-335 (even)).

EDX points to a buffer with the 42 bytes of the teletext packet (excluding the clock run-in and the framing code).

ChangeCallback()

Purpose:	Called by the VBI decoder when the BT848.VXD signals that either the channel or the video format is changed. The parameters are as described for the corresponding call-back function in the BT848.VXD.

# Appendix C: Bt848A/Bt849 Features

## I. Introduction

This document is intended to describe the changes for Bt848A implementation.

Analog video format PAL-NC is supported in Bt848A. Currently two digital camera, Silicon Vision, Inc. Model 512N and Logitech VideoMan 1.3, are supported in this release of the driver. Detailed specifications of the cameras are listed under section Camera Specifications in this documentation.

## II. Implementation Changes

### A. Bt848Dlg.dll

Bt848Dlg.dll consists of the APIs that are exported in the Windows environment such that other applications can call these functions as a mean to communicate with the hardware.

A new enumerated value, Err_WrongHardware, has been added to type ErrorCode to indicate wrong hardware has been installed in the system; i.e. calling a function either designed for Bt848A or with a parameter for Bt848A only. Among existing Bt848 functions, invoking SetConnector( ConCamera ) or SetVideoFormat( VFormat_PAL_NC ) will result in return value Err_WrongHardware when only Bt848 is installed.

Additional APIs added in Bt848Dlg.dll for Bt848A are listed as follow:

#### ErrorCode SetSignalFormat( SignalFormat sFormat )

Purpose:	Select input signal format

Input:	SignalFormat sFormat - video signal format

Signal_Analog	- Analog

Signal_CCIR656	- CCIR 656

Signal_ByteStream	- ByteStream

Signal_ExtHSYNC_VSYNC	- External HSYNC, VSYNC

Signal_ExtHSYNC_FIELD	- External HSYNC, FIELD

Output:	None

Return:	Success, Fail or Err_WrongHardware

#### SignalFormat GetSignalFormat( void )

Purpose:	Obtain input signal format

Input:	None

Output:	None

Return:	SignalFormat - video signal format

Signal_Analog	- Analog

Signal_CCIR656	- CCIR 656

Signal_ByteStream	- ByteStream

Signal_ExtHSYNC_VSYNC	- External HSYNC, VSYNC

Signal_ExtHSYNC_FIELD	- External HSYNC, FIELD

#### ErrorCode SetSyncVideoRef( SyncVideoRef syncRef )

Purpose:	Select sync video reference

Input:	SyncVideoRef syncRef - sync video reference

VRef_alignedCb	- HS/VS aligned with Cb

VRef_alignedY0	- HS/VS aligned with Y0

VRef_alignedCr	- HS/VS aligned with Cr

VRef_alignedY1	- HS/VS aligned with Y1

Output:	None

Return:	Success, Fail or Err_WrongHardware

#### SyncVideoRef GetSyncVideoRef( void )

Purpose:	Obtain sync video reference

Input:	None

Output:	None

Return:	SyncVideoRef syncRef - sync video reference

VRef_alignedCb	- HS/VS aligned with Cb

VRef_alignedY0	- HS/VS aligned with Y0

VRef_alignedCr	- HS/VS aligned with Cr

VRef_alignedY1	- HS/VS aligned with Y1

#### ErrorCode SetDecInputClock( DecInputClk iClk )

Purpose:	Select decoder input clock

Input:	DecInputClk iClk - decoder input clock

DecClk_XTAL	- normal XTAL0/XTAL1

DecClk_PLL	- PLL

DecClk_GPCLK	- GPCLK

DecClk_invGPCLK	- GPCLK- inverted

Output:	None

Return:	Success, Fail or Err_WrongHardware

Note:	When either GPCLK or GPCLK-inverted is selected, a call to SetPLL() is desirable such that if the digital camera cord is disconnected, blue-field timing will still run properly.

#### DecInputClk GetDecInputClock( void )

Purpose:	Obtain decoder input clock

Input:	None

Output:	None

Return:	DecInputClk iClk - decoder input clock

DecClk_XTAL	- normal XTAL0/XTAL1

DecClk_PLL	- PLL

DecClk_GPCLK	- GPCLK

DecClk_invGPCLK	- GPCLK- inverted

#### ErrorCode SetOutputClock( GPCLKOutputClk oClk )

Purpose:	Select GPCLK output clock

Input:	GPCLKOutputClk oClk - GPCLK output clock

GPCLK_CLKx1	- pixel rate

GPCLK_Xtal0	- XTAL

GPCLK_PLL	- PLL

GPCLK_invPLL	- PLL - inverted

Output:	None

Return:	Success, Fail or Err_WrongHardware

#### GPCLKOutputClk GetOutputClock( void )

Purpose:	Obtain GPCLK output clock

Input:	None

Output:	None

Return:	GPCLKOutputClk - GPCLK output clock

GPCLK_CLKx1	- pixel rate

GPCLK_Xtal0	- XTAL

GPCLK_PLL	- PLL

GPCLK_invPLL	- PLL - inverted

#### ErrorCode SetEnableSyncOutput( State state )

Purpose:	Enable/Disable syncs output for synchronizing video input

Input:	State state - On to enable; Off to disable

Output:	None

Return:	Success, Fail or Err_WrongHardware

#### State GetEnableSyncOutput( void )

Purpose:	Check whether if syncs output for synchronizing video input is enabled

Input:	None

Output:	None

Return:	State - On for enable; Off for disable

#### ErrorCode SetTimGenByte( TimeGenByte tgb )

Purpose:	Load time generator bytes into SRAM

Input:	TimeGenByte tgb - type of timing generator bytes to be loaded

TGB_NTSC624 - NTSC signal with 624 total pixels

TGB_NTSC780 - NTSC signal with 780 total pixels

TGB_NTSC858 - NTSC signal with 858 total pixels

Output:	None

Return:	Success, Fail or Err_WrongHardware

#### TimeGenByte GetTimGenByte( void )

Purpose:	Obtain wich time generator bytes has been loaded into SRAM

Input:	None

Output:	None

Return:	TimeGenByte - type of timing generator bytes loaded

TGB_NTSC624 - NTSC signal with 624 total pixels

TGB_NTSC780 - NTSC signal with 780 total pixels

TGB_NTSC858 - NTSC signal with 858 total pixels

#### ErrorCode SetHReset( State state )

Purpose:	Enable/Disable HRESET to be shortened from 64 pixels to 32 pixels

Input:	State state - On to enable; Off to disable

Output:	None

Return:	Success, Fail or Err_WrongHardware

#### State GetHReset( void )

Purpose:	Check whether if HRESET has be shortened from 64 pixels to 32 pixels

Input:	None

Output:	None

Return:	State - On for enable; Off for disable

#### ErrorCode SetPLL( PLLFreq pll )

Purpose:	Set PLL to synthesize desired frequency

Input:	PLLFreq pll - desired synthesized frequency

NoPLL	- not using PLL

KHz28636	- 28.63636 MHz

KHz35469	- 35.46895 MHz

KHz27000	- 27.00000 MHz

KHz24545	- 24.54545 MHz

KHz29500	- 29.50000 MHz

KHz19636	- 19.63636 MHz

Output:	None

Return:	Success, Fail or Err_WrongHardware

Note:	This function is used to initialize PLL to run at a desired synthesized frequency. Since PLL can go out of lock after initializing, a wait of up to 100ms is implemented to ensure PLL out of lock bit is not set.

#### PLLFreq GetPLL( void )

Purpose:	Obtain what synthesized frequency has been set to PLL

Input:	None

Output:	None

Return:	PLLFreq pll - desired synthesized frequency

NoPLL	- not using PLL

KHz28636	- 28.63636 MHz

KHz35469	- 35.46895 MHz

KHz27000	- 27.00000 MHz

KHz24545	- 24.54545 MHz

KHz29500	- 29.50000 MHz

KHz19636	- 19.63636 MHz

#### ErrorCode SetVideoInfo( VSIZEPARMS vSize )

Purpose:	Set video size parameters

Input:	VSIZEPARMS vSize - parameters to set video size

WORD HActive	- number of active horizontol pixels

(Clkx1_HACTIVE)

WORD HDelay	- number of pixels delayed before actual signal

(Clkx1_HDELAY)

WORD VActive	- VActive register value

WORD Vdelay	- VDelay register value

WORD linesPerField	- number of active lines in each field

Output:	None

Return:	Success, Fail or Err_WrongHardware

#### ErrorCode GetVideoInfo( VSIZEPARMS * vsize )

Purpose:	Obtain video size parameters

Input:	None

Output:	VSIZEPARMS * vsize - pointer to video size parameters obtained

WORD HActive	- number of active horizontol pixels

(Clkx1_HACTIVE)

WORD HDelay	- number of pixels delayed before actual signal

(Clkx1_HDELAY)

WORD VActive	- VActive register value

WORD VDelay	- VDelay register value

WORD linesPerField	- number of active lines in each field

Return:	Success or Fail

#### ErrorCode ResetFieldCounter( void )

Purpose:	Reset field counter

Input:	None

Output:	None

Return:	Success, Fail or Err_WrongHardware

#### int GetFieldCounter( void )

Purpose:	Obtain field counter

Input:	None

Output:	None

Return:	Field counter

#### int GetHWRevisionID( void )

Purpose:	Obtain Bt848/Bt848A revision ID

Input:	None

Output:	None

Return:	Revision ID if Bt848/Bt848A is found; else -1

#### BOOL IsABt848A( void )

Purpose:	Check if a Bt848A hardware is installed

Input:	None

Output:	None

Return:	TRUE if found, else FALSE

## III. Windows Registry Key

The following are the Windows registry keys maintained by the Bt848A. Descriptions to each key and the possible values are also listed.

| Registry Key | Descriptions |
| --- | --- |
| CameraIndex | Camera index in video source dialog 0 - 1st camera 1 - 2nd camera etc. |
| connector | Which connector is used for video input 0 - Camera 1 - Svideo 2 - Tuner 3 - Composite |
| HActive | Number of active horizontal pixels |
| HDelay | Number of horizontal pixels delayed before active pixels |
| HReset | Enable/Disable HRESET to be shortened from 64 pixels to 32 pixels 1 - Enable 0 - Disable |
| InClk | Select decoder input clock 0 - Normal Xtal0/Xtal1 mode 1 - PLL 2 - GPCLK 3 - GPCLK inverted |
| LinesPerField | Number of lines in each field |
| OutClk | Select GPCLK output clock 0 - CLKx1 1 - Xtal0 2 - PLL 3 - PLL inverted |
| PLLFreq | Synthesized frequency 0 - Not using PLL 1 - 28.63636 MHz 2 - 35.46895 MHz 3 - 27.00000 MHz 4 - 24.54545 MHz 5 - 29.50000 MHz 6 - 19.63636 MHz |
| RefFreq | Reference frequency on board 28 - 28.63636 MHz 35 - 35.46895 MHz |
| SignalFormat | Input signal format 0 - Analog 1 - CCIR 656 2 - ByteStream 3 - External HSYNC, VSYNC 4 - External HSYNC, FIELD |
| SyncOutput | Enable/Disable syncs output for synchronizing video input 1 - Enable 0 - Disable |
| SyncRef | Sync video reference 0 - HS/VS aligned with Cb 1 - HS/VS aligned with Y0 2 - HS/VS aligned with Cr 3 - HS/VS aligned with Y1 |
| TimGenByte | Timing Generator Load Byte 0 - NTSC 624 1 - NTSC 780 2 - NTSC 858 |
| VActive | Number of active vertical lines |
| Vdelay | Number of vertical pixels delayed before active pixels |

## IV. Interfacing with Digital Camera

When a digital camera is selected as video input source, certain APIs are required to invoke in order to setup the hardware accordingly. The APIs are closely related to the structure which contains the specifications of the camera. The structure is defined as follow:

typedef struct tag_cameraSpec

{

SignalFormat signal;              // which digital signal format the camera has

VideoFormat    vidFormat;        // video standard

SyncVideoRef syncRef;            // which sync video reference is used

State                syncOutput;      // enable sync output for sync video input?

DecInputClk    iClk;                  // which input clock is used

TimeGenByte    tgb;                    // which timing generator byte does the camera use

State                HReset;              // uses 64 pixels(Off) or 32 pixels(On) for HReset

PLLFreq            pllFreq;            // what synthesized frequency to set PLL to

VSIZEPARMS      vSize;                // video sizes the camera produces

} CameraSpec;

A camera specification will then be initialized as:

CameraSpec spec = { … };

The corresponding APIs required to be invoked are:

SetConnector( ConCamera, TRUE/FALSE );

SetSignalFormat( spec.signal );

SetVideoFormat( spec.vidFormat );

SetSyncVideoRef( spec.syncRef );

SetEnableSyncOutput( spec.syncOutput );

SetTimGenByte( spec.tgb );

SetHReset( spec.HReset );

SetPLL( spec.pllFreq );

SetDecInputClock( spec.iClk );

SetVideoInfo( spec.vSize );

On the other hand, if analog signal is desired as source input, then APIs should be called in this fashion:

SetConnector( ..., TRUE/FALSE );

SetSignalFormat( Signal_Analog );

SetVideoFormat( ... );

## V. Camera Specifications

In order to support different digital camera models, the following is a list of the required parameters to initialize the hardware. Parameters for the two digital cameras supported are provided for reference.

| Parameter | Silicon Vision Inc. Model 512N | Logitech VideoMan 1.3 |
| --- | --- | --- |
| Signal Format | CCIR 656 | CCIR 656 |
| Video Format | NTSC-M | NTSC-M |
| Sync Video Ref | aligned with Cb | aligned with Cb |
| Decoder Input Clock Select | GPCLK | GPCLK |
| Enable Sync Output for Sync video input | Off | Off |
| Timing Generator Load Byte | NTSC 624 pixels | NTSC 780 pixels |
| PLL synthesized frequency | 19.63636 MHz | 24.54545 MHz |
| HSFMT | Off | Off |
| Video Parameters: HActive | 512 | 640 |
| HDelay | 0x64 | 0x80 |
| VActive | 480 | 480 |
| Vdelay | 0x13 | 0x1A |
| LinesPerField | 240 | 240 |

## VII. Supporting Tools

Several tools are developed in order to support the Bt848A and the supporting camera models. They are listed in the following.

### A. Bt848A Register Tool

This tool provides a complete read/write interface to all the registers in Bt848A. User can easily modifies any registers that have write permission by editing the field associated with any register. Frequent reading of the registers in Bt848A is implemented such that up-to-date contents of all registers are showing at all time.

### B. SRAM Load/Dump Utility

A utility program allows reading and displaying the SRAM contents. In addition, loading the SRAM content can be selected from the choice of a fixed value, ascending values, or content in a file.

### C. Logitech VideoMan Camera Tool

This register tool is developed to act as an interface to the Logitech VideoMan cameras. Models VideoMan 1.2 and 1.3 are supported in this tool although some controls are only applicable to version 1.3. It uses I2C protocol to communicate with the camera.

### D. Silicon Vision Camera Configuration Tool

A configuration utility is provided by Silicon Vision, Inc. to select many features available to the camera.

Registry key connector already exists in Bt848. Added new value of 0 to denote using camera as video source.